Q1.	I have been using a SanDisk FlashDisk on an HP95LX.  I am going to upgrade to an 	HP200LX (or an HP100LX).   What will I need to do in order for my new palmtop 	to recognize my SanDisk card?

A1.	Both the HP100LX and the HP200LX will automatically recognize all SanDisk 	FlashDisks.  HP100LX/200LX have built-in support for all SanDisk FlashDisks, so 	there is no need to load any special drivers to get it working.

If you are using Stacker on your FlashDisk, then you will need to load the Stacker 	drivers onto your HP100LX/200LX Palmtop.  Refer to question Q14 for this 	procedure.

Q2.	What SanDisk products work on the HP95LX palmtop?

A2.	SDP5-XX

SDPL5-XX

SDP5A-XX (unless XX=80, 110 or 175, because these are Type III cards)

SDP3B-XX

Note: SDP-XX will work, but only as read-only cards.

Q3.	What SanDisk products work on the HP100LX/200LX palmtop?

A3.	SDP-XX

SDP5-XX

SDPL5-XX

SDP5A-XX (unless XX=80, 110 or 175, because these are Type III cards)

SDP3B-XX

Q4.	Will the SD-XX series of FlashDisk cards work in the HP 95LX/100LX/200LX 	palmtop?

A4.	No, because the SD-XX series is not PCMCIA-ATA compatible.

Q5.	I am having trouble using a stacked 20MB SanDisk card on the HP 95LX.

A5.	This will not work.  This is due to the version of DOS that is shipped with the 	HP95LX palmtop, MS-DOS version 3.2.  On DOS versions 3.3 and below, the 	maximum capacity of a mass storage device partition is 32 MBytes, even if this is 	due to Stacker or some other data compression software.

Q6.	Do the HP Palmtops support Hot Insertion?

A6.	Refer to the following table:

| Palmtop Model | Hot Insertion Supported? |
| --- | --- |
| HP95LX | YES |
| HP100LX | NO |
| HP200LX | NO |

Q7.	I am getting an error message RAM4 bad Plug in Card when I run the hardware test 	on my HP95LX/100LX/200LX.

A7.	This is associated with the HP95LX and not the SanDisk card.  There is nothing 	wrong with the SanDisk card, so the error message can be ignored.  In fact, all 	SanDisk cards give this error message (the test was designed to work with SRAM 	cards).

Q8.	Why can’t I access my SanDisk card on the HP95LX?  When I try to go to drive A:, 	all I get is an error message “I/O Error accessing device.”

A8.	You need to install (or reinstall) the SanDisk driver onto your HP95LX (see next 	question).

Q9. 	How do I configure my HP95LX to recognize my SanDisk Card?

A9.	You must install the SanDisk device driver installation program SDPSTART.COM 	onto the HP95LX. Note: Since the HP95LX does not have a floppy disk drive, you 	must use the Driver Installation ROM Card to transfer the SDPSTART.COM 	program to your HP95LX.  The Driver Installation ROM Card looks like the 	SanDisk Flash Card, and it probably came with your SanDisk Card when the card 	was purchased.  If it did not, then you must use the HP Connectivity Kit in order to 	transfer the SDPSTART.COM file (contained on your SanDisk SDP Installation 	Diskette) from a desktop computer to your HP95LX.  If this is the case, refer to the 	instructions in the HP Connectivity Kit Manual on how to transfer the 	SDPSTART.COM file to your palmtop.

1.	Close all open applications on your HP95LX.  (Failure to do so may result in 				loss of data when your HP95LX reboots to load the driver).

2.	Insert the Driver Installation ROM Card with the SanDisk logo face up into your 				HP95LX card slot.

3.	Press <FILER>.

4.	Hit the <F5> key and type A: <ENTER> to go to the A: drive.

5.	Move the cursor to the SDPSTART.COM program and hit <F4> to run this 				program.

6.	Follow the instructions on your HP95LX screen to complete the installation.

The HP95LX should now recognize your SanDisk Card. (If you are using  Stacker, 	however, you must still load the Stacker driver onto your HP95LX).  When you 	have completed the installation, you should have the following file:

SDPDRV.SYS 	located in the C:\SDP directory on your HP95LX.

In addition, the following line should appear in you CONFIG.SYS file located on the 	root directory of your HP95LX’s C: drive.

DEVICE=C:\SDP\SDPDRV.SYS	line in your CONFIG.SYS file in the C:\ 		root directory.

Q10. How do I configure my HP100LX/200LX to recognize my SanDisk Card?

A10.	Both the HP100LX and the HP200LX have built-in support for SanDisk Cards, 	which means it is automatically recognized by the palmtop.  There are no installation 	programs or drivers to load in order to access your SanDisk Card.  If you are using 	the Stacker option, however, you must configure your palmtop for Stacker by 	loading the Stacker device driver onto the HP palmtop.

Q11.	How do I reformat my SanDisk card on the HP95LX?

A11.	To format your SanDisk Card on the HP95LX, you will need the two utilities

provided with your SanDisk Card, SDISK and SFORMAT, to partition and

format your SanDisk Card.

1.	Back up any important data on your card to another source prior to

reformatting your SanDisk card.

2.	Go to DOS by pressing <FILER> then <MENU> and select SYSTEM and 				press <ENTER>.  If you have Stacker pre-loaded on your SanDisk Card, go to

step 3.  If you are not using Stacker, proceed to step 6.

3.	At the DOS C> prompt type CD\STACKER and press <ENTER>

4.	Type STACKER -A: and press <ENTER>

5.	If you plan on reinstalling Stacker on your SanDisk card, then you may

now want to back up the Stacker files on your SanDisk card to the C:

drive of the HP95LX.  If you do not back up these Stacker files, then

you may still use the Stacker Utility Diskette that came with your

SanDisk card when you purchased it with the Stacker option.  Even if

you have this diskette, it will still be much easier to restack your

SanDisk Card with these files already stored on your HP95LX, instead of

transferring them to your HP95LX from the Stacker Utility Diskette.

To back up your Stacker files, at the C> prompt type MD\SBACKUP and

press <ENTER>.  Then type COPY A:\*.* C:\SBACKUP and press 					<ENTER>

6.	Type CD\SDP and press <ENTER>

7.	Type SDISK A: and press <ENTER> to partition your card.

8.	Type SFORMAT A: and press <ENTER> to format your card.

Q12.	How do I reformat my SanDisk card on the HP100LX/200LX?

A12.	To format your SanDisk Card on the HP100LX/200LX:

1.	Back up any important data on your card to another source prior to

reformatting your SanDisk card.

2.	Go to DOS by pressing <FILER> then press O then D.

If you have Stacker pre-loaded on your SanDisk Card, go to

step 3.  If you are not using Stacker, proceed to step 6.

3.	At the DOS C> prompt type CD\STACKER and press <ENTER>

4.	Type STACKER -A: and press <ENTER>

5.	If you plan on reinstalling Stacker on your SanDisk card, then you may

now want to back up the Stacker files on your SanDisk card to the C:

drive of the HP100LX/200LX.  If you do not back up these Stacker files, then

you may still use the Stacker Utility Diskette that came with your

SanDisk card when you purchased it with the Stacker option.  Even if

you have this diskette, it will still be much easier to restack your

SanDisk Card with these files already stored on your HP100LX/200LX, instead 				of transferring them to your HP100LX/200LX from the Stacker Utility Diskette.

To back up your Stacker files, at the C> prompt type MD\SBACKUP and

press <ENTER>.  Then type COPY A:\*.* C:\SBACKUP and press 					<ENTER>

6.	Type FDISK100 A: and press <ENTER> to partition your card.

7.	Type FORMAT A: and press <ENTER> to format your card.

Q13.	How do I install (or reinstall) Stacker on my HP95LX?

A13.	Even though your SanDisk Card may have come with Stacker pre-loaded on the 	card, you must still configure your HP95LX palmtop to recognize Stacker by 	copying and loading the Stacker driver onto your HP95LX.  Two methods are 	shown below, one using the SINSTALL program and the other by manually copying 	and loading the necessary files through DOS commands.

Method #1--Installing Stacker using SINSTALL.EXE (easiest method)

1.	Insert your SanDisk Flash Card into the HP95LX palmtop.

2.	Get into the filer application

Type <FILER>

3.	Go to the A: drive.

Hit the <F5> key and type A: <ENTER>

4.	Run the SINSTALL installation program.

Move the cursor to the SINSTALL.EXE (or SINSTALL.BAT) file and hit 				<F4>

5.	Follow the instructions on your HP95LX screen to complete the Stacker 					installation.

Method #2--Installing Stacker Manually

1.	Insert your SanDisk Flash Card into the HP95LX palmtop.

2.	Get into the filer application

Type <FILER>

3.	Go to DOS.

Press <MENU> and select SYSTEM and press <ENTER>

4.	Make a C:\STACKER directory.

Type MD\STACKER <ENTER>.  If you see a “Directory already exists” error 			message do not worry--ignore it.

5.	Copy the Stacker driver from the A: drive into the C:\STACKER directory and 				rename the file to its proper filename (rename STACKER.DRV to 					STACKER.COM).

Type COPY A:\STACKER.DRV C:\STACKER\STACKER.COM 					<ENTER>

6.	Append the Stacker statement to the CONFIG.SYS file of the C: drive’s root `				directory.

Type COPY C:\CONFIG.SYS+A:\SINSTAL1.TXT C:\CONFIG.SYS 				<ENTER>

7.	Reboot the HP95LX

Hit <CTRL> <ALT> <DEL> simultaneously.

Q14.	How do I install (or reinstall) Stacker on my HP100LX/200LX?

A14.	Even though your SanDisk Card may have come with Stacker pre-loaded on the 	card, you must still configure your HP100LX/200LX palmtop to recognize Stacker 	by copying and loading the Stacker driver onto your HP100LX/200LX.  Two 	methods are shown below, one using the SINSTALL program and the other by 		manually copying and loading the necessary files through DOS commands.

Method #1--Installing Stacker using SINSTALL.EXE (easiest method)

1.	Insert your SanDisk Flash Card into the HP100LX/200LX palmtop.

2.	Get into the filer application

Type <FILER>

3.	Go to the A: drive.

Hit the <F5> key and type A: <ENTER>

4.	Run the SINSTALL installation program.

Move the cursor to the SINSTALL.EXE (or SINSTALL.BAT) file and hit 				<F4>

5.	Follow the instructions on your HP100LX/200LX screen to complete the 				Stacker installation.

Method #2--Installing Stacker Manually

1.	Insert your SanDisk Flash Card into the HP100LX/200LX palmtop.

2.	Get into the filer application

Type <FILER>

3.	Go to DOS.

Press <MENU> and press O and then press D

4.	Make a C:\STACKER directory.

Type MD\STACKER <ENTER>.  If you see a “Directory already exists” error 			message do not worry--ignore it.

5.	Copy the Stacker driver from the A: drive into the C:\STACKER directory and 				rename the file to its proper filename (rename STACKER.DRV to 					STACKER.COM).

Type COPY A:\STACKER.DRV C:\STACKER\STACKER.COM 					<ENTER>

6.	Copy the CONFIG.SYS from the D: drive to the C: drive.

Type COPY D:\CONFIG.SYS C:\ <ENTER>

7.	Copy the AUTOEXEC.BAT from the D: drive to the C: drive.

Type COPY D:\AUTOEXEC.BAT C:\ <ENTER>

8.	Append the Stacker statement to the CONFIG.SYS file of the C: drive’s root 				directory.

Type COPY C:\CONFIG.SYS+A:\SINSTAL1.TXT C:\CONFIG.SYS 				<ENTER>

9.	Reboot the HP100LX/200LX.

Hit <CTRL> <ALT> <DEL> simultaneously.

Q15.	How can I look at my “uncompressed” files on my Flash Card 	(HP95LX/100LX/200LX)?

A15.	If you are using Stacker, it is possible to look at your Flash Card in either its 	“compressed” mode or its “uncompressed” mode.  If you are currently in the Stacker 	compressed mode, then you must “unmount” Stacker in order to see your card in the 	uncompressed mode.  To do this:

1.	Go to DOS.

2.	At the DOS C> prompt, type CD\STACKER <ENTER>

3.	Type STACKER -A <ENTER>

4.	Type A: <ENTER>

5.	Type DIR <ENTER>

You should now see your files in the uncompressed portion of your card.

To return to the “compressed” portion of your Flash card:

1.	Type C: <ENTER>

2.	Type CD\STACKER <ENTER>

3.	Type STACKER +A <ENTER>

4.	Type A: <ENTER>

5.	Type DIR <ENTER>