Introduction

This document explains how to build a control simulation under

GAF.  A brief description of basic concept and steps to build a

simulation is presented.  Then, a simple example of water heater

control is used to explore the basics of GAF.

What is a segment

A segment is a text file contains:

variable declarations (IN, OUT, INOUT, and LOCAL)

condition to enable the segment

reset

post-processing

control segment(s)

simulation (or feedback) segment(s)

calculated by other segment (as OUT or INOUT)

application constants, can be modified through interactive

interface during simulation

OUT variables

OUT variables are calculated outputs by this segment.  OUT

variables are either:

be used by other segment as IN variables

evaluation result

Note that for each output variable, there must be only one

segment declares it as an OUT or INOUT variable.  If more

than one segment declares an output variable as OUT or INOUT,

the result is undefined.

INOUT variables

INOUT variables are both referenced and modified by this

segment, i.e., it is the combination of both IN and OUT

variables.  Normally INOUT variables are referenced by this

segment and other segments.

All of the IN, OUT, and INOUT variables are visible to other

segments, i.e. other segments can use this segment's OUT as its

IN.

LOCAL variables

LOCAL variables are local to this segment.  Normally, local

variables are used to store temporary values so it can be

used by fuzzy rules or other statements within the segment.

2. Define fuzzy membership

The second step would be define the fuzzy membership set.  GAF

uses four points for each set.  In general, fuzzy sets should

be defined for each variable, either in out or local, that is

used in the fuzzy rules.  For example, as described in the

following figure.  A simple way to define membership sets for a

variable is to equaly divide the range of the variable into six

to obtain five sets - very low (VL), low (LO), medium (M), high

(HI), and very high (VH).

-----     -     -     -     -----

\   / \   / \   / \   /

\ /   \ /   \ /   \ /

X     X     X     X

/ \   / \   / \   / \

/   \ /   \ /   \ /   \

+-----+-----+-----+-----+-----+-----+

-1  -0.67 -0.33   0    0.33  0.67   1

| very low  |  medium   | very high |

|    low    |   high    |

As the above diagram shows these five membership sets are

symmatrically divided.  Then, use these membership sets for the

starting point.

SYMBOL very_low  (-1.00, -1.00, -0.67, -0.33, 1.0)

SYMBOL low       (-0.67, -0.33, -0.33   0,    1.0)

SYMBOL medium    (-0.33,  0,     0,     0.33, 1.0)

SYMBOL high      ( 0,     0.33,  0.33,  0.67, 1.0)

SYMBOL very_high ( 0.33,  0.67,  1,     1,    1.0)

3. Translate ideas into fuzzy rules

Defining fuzzy rules is quite straight forward.  Basically,

this step is simply converting an imprecise idea of how things

should work into fuzzy rules.  For example, based on common

sense to control a water heater, a simple rule may be like:

"if water usage is heavy and water is cold,

then the gas fuel valve to heat the tank must be full open"

This idea can turn into a rule like:

IF water_usage IS heavy AND water IS cold

THEN fuel_valve IS full_open

Where water_usage and water are IN variables, fuel_valve is an

OUT variable; heavy, cold, and full_open are membership sets

defined for corresponding variables.  Note that all variables

that are part of "THEN" statement must be declared as OUT or

INOUT.

Please refer to step 6 for more information.

4. Initializes the segment

Next step is to define what do you want the segment to be

initialized.  In general, the initialization of a segment is to

initialize all outputs (OUT and INOUT variables) and its local

variables.  The initialization will be executed when the system

"comes up" (i.e., the first time it runs the segment) or

requested from user interactive interface.  The initialization

formulas and other processing formulas are arranged like this:

what's in the text      remarks

-------------------     ----------------------------------------

INITIALIZATION          must have this key word

... ;             statements for initialization

each statement must end with ';'

RESET                   optional key word for reset processing

... ;           optional statements for reset processing

PRE_PROCESSING          optional key word for pre-processing

... ;           optional statements for pre-processing

POST_PROCESSING         optional key word for post-processing

... ;           optional statements for post-processing

END;                    must have this key word (with ';')

Note that INITIALIZATION must be the first statemant and END

must be the last statement.  For example, for the above water

heat control, the segment should initialize the fuel_valve to

close.

INITIALIZATION

fuel_valve = 0;

PRE_PROCESSING

...

END;

5. Reset processing

Similar to initialization, the reset processing is to reset the

segment when the segment is disabled.  The reset processing is

only necessary when it is different from the initialization.

6. Pre-processing and post-processing

The pre-processing is a set of statements to be calculated

before the fuzzy rules are inferenced.  The post-processing is

a set of statements to be calculated after the fuzzy rules are

inferenced.  The formulas can be used for calculating

references or other simple mathematical models.  Pre-processing

is calculated before any fuzzy rule been evaluated, so the

results of these formulas may be used by fuzyy rules to

inference the result.

7. Segment enable condition

Now, it's time to decide how and when should this segment be

enabled.  The enable condition determines whether the segment

shoud run or not.  If enable condition is not specified GAF

treats the segment as enabled all the time.

Example: Water Heater Temperature Control

Let's use a simple water heater control to illustrate these steps

in constructing a control segment.  First, let's explain the

simple device that we are going to control.

The water heater

The following diagram shows the physical layout of the water

heater.

temperature sensor

!

!

/-------!-------\     valve

high     |       !      <=======x======= water in

sensor----x      !       |

|       !       |

|       !       |

|       !       |

|       !       |

|       !       |

|       !       |

|       !       |

|       !       |   low

|       !      x----sensor

water   valve   |               |

out <=====x======               |

\---------------/

^^^^^^^^^    burner

!

!<======x=======fuel

valve

As shown in the diagram, these are the major devices for the

water heater and can be used by the controller to control the

water temperature.

2 water valves: in & out

2 water sensors: high & low

1 water temperature sensor

1 fuel valve

Let's also assume that the heater can heat a full tank of water

up one degree per second with max gas fuel.  The water tank can

be drained in 100 seconds with full-open water out valve.

To simplify the problem, let's split the controller into two

part: a water temperature control and a water volume control.

The reason, besides cover this example in few pages, is to

isolate the temperature controller from the water controller.

So, inside the temperature controller we can assume that the

water volume is a constant.

Step 1: Define variables

Since we are only controling the water temperature, it is quite

easy to identify the output of the controller, i.e., the fuel

valve.  The inputs to this controller are: the water

consumption (from the valve of water out), the water

temperature from the sensor.  Also, there should be a target

temperature as an input to this controller.  After identify

these variables, now we can find out their range and declare

these variables.

IN Target_Temp (32, 212)    ! target temperature

IN Temperature (32, 212)    ! measured temperature

IN Water_out (0, 1)       ! 0 - 1 input from the water out valve

OUT Gas_valve (0, 10)     ! 0 - 10 volts output to the fuel valve

Step 2: Define Pre-processing and Initialization

Because we only care about the temperature error, which is the

difference between the reference and the measured water

temperature.  So, we can use the pre-processing to calculate

the temperature error before inference the fuzzy rules.  In

order to do this, a new local variable has to be put in:

LOCAL Temp_error (-180, 180)

Then the pre-processing and initialization is:

INITIALIZATION

Gas_valve = 0;    ! turn off the valve

Temp_error = 0;   !

PRE_PROCESSING

Temp_error =  Temperature - Target_Temp ;

END;

Step 3: Define Fuzzy Membership

Let's start defining membership set by using the simple method

discussed.  Note that for the water consumption, and gas valve

only the right half (> 0) is used.

The Water_out  variable divided into 5 bands, i.e., very heavy,

heavy, medium, light, and zero comsumption.  The result would

be:

-     -     -     -     -------

\   / \   / \   / \   /

\ /   \ /   \ /   \ /

X     X     X     X

/ \   / \   / \   / \

/   \ /   \ /   \ /   \

+-----+-----+-----+-----+-----+

0    0.2   0.4   0.6   0.8   1.0

!                               below     low    high   above  truth

SYMBOL Zero      OF Water_out (   0.0,    0.0,    0.0,   0.2,    1.0)

SYMBOL Light     OF Water_out (   0.0,    0.2,    0.2,   0.4,    1.0)

SYMBOL Medium    OF Water_out (   0.2,    0.4,    0.4,   0.6,    1.0)

SYMBOL Heavy     OF Water_out (   0.4,    0.6,    0.6,   0.8,    1.0)

SYMBOL VeryHeavy OF Water_out (   0.6,    0.8,    1.0,   1.0,    1.0)

The Temp_error variable is divided into these five membership

sets (with extended VeryHot and VeryCold):

-------     -     -     -     -----

\   / \   / \   / \   /

\ /   \ /   \ /   \ /

X     X     X     X

/ \   / \   / \   / \

/   \ /   \ /   \ /   \

+-..--+-----+-----+-----+-----+--..-+

-180   -40   -20    0    20    40    180

!                               below     low    high   above   truth

SYMBOL VeryCold OF Temp_error (-180.0, -180.0,  -40.0,  -20.0,    1.0)

SYMBOL Cold     OF Temp_error ( -40.0,  -20.0,  -20.0,    0.0,    1.0)

SYMBOL OnTarget OF Temp_error ( -20.0,    0.0,    0.0,   20.0,    1.0)

SYMBOL Hot      OF Temp_error (   0.0,   20.0,   20.0,   40.0,    1.0)

SYMBOL VeryHot  OF Temp_error (  20.0,   40.0,  180.0,  180.0,    1.0)

The Gas_valve variable:

-     -     -     -     -------

\   / \   / \   / \   /

\ /   \ /   \ /   \ /

X     X     X     X

/ \   / \   / \   / \

/   \ /   \ /   \ /   \

+-----+-----+-----+-----+-----+

0     2     4     6     8    10

!                              below     low    high   above   truth

SYMBOL Off      OF Gas_valve (   0.0,    0.0,    0.0,    2.0,   1.0)

SYMBOL VeryLow  OF Gas_valve (   0.0,    2.0,    2.0,    4.0,   1.0)

SYMBOL Low      OF Gas_valve (   2.0,    4.0,    4.0,    6.0,   1.0)

SYMBOL High     OF Gas_valve (   4.0,    6.0,    6.0,    8.0,   1.0)

SYMBOL VeryHigh OF Gas_valve (   6.0,    8.0,   10.0,   10.0,   1.0)

Step 4: Translate Ideas Into Fuzzy Rules

Using common sense to implement rules, such as turn this idea

if the water consumption is very heavy and the water is very

cold then full open the gas fuel valve

into a rule

IF Water_out IS VeryHeavy AND Temp_error IS VeryCold

THEN Gas_valve IS VeryHigh

There is an easy approach for this simple controller - by

filling a table.  As shown in the following table, this rule's

output VeryHigh is in the top left cell.  The rest of the table

can be filled in based on this kind of common ideas.

VeryHeav  Heavy     Medium    Light     Zero

--------  --------  --------  --------  --------

VeryCold    VeryHigh  VeryHigh  VeryHigh  VeryHigh  VeryHigh

Cold        VeryHigh  VeryHigh  VeryHigh  High      High

OnTarget    VeryHigh  High      Low       VeryLow   Off

Hot         High      Low       VeryLow   Off       Off

VeryHot     Low       VeryLow   Off       Off       Off

Note that some may argue that the "table filling" approach can

be applied not just simple controls but also complex controls

by pairing two inputs and one output.  But a system built with

this approach (without optimization) may require intense CPU

power and contain extremely complicated rule set.

Step 5: Segment Enable Condition

In order to avoid unnecessary sensitive reaction from the

controller, a hysteresis null band can be put in to enable or

disable the control segment.   As in the example file

heattank.sim, the output variable Heating is calculated based

on a hysteresis null band.  So, we can add an ENABLE condition

to achieve this:

ENABLE = Heating;

This will disable the segment within the hysteresis null band.

When the segment is disabled we want shut off the output

Gas_valve, because this is identical to the initialization, so

it's not necessary to define the RESET processing.

Results

Please refer to the accompany files heattank.x01 and

heattank.x02 for the result of this excercise.

Note:

1.  The Temp_error is calculated outside this control segment.

2.  The symbol Off and VeryHigh defined for Gas_valve have special

"center" values, which define the output center value for these

fuzzy sets.

3.  The .x02 has incorporated some simplifications of the rules

(from the above rule table).

The heattank.fbk emulates the reaction of the water tank with

simple calculation based on the input/output energy.  A few

rules are added to introduce some heat loss and non-linear

portion of the system.