:gdoc sec='IBM Internal Use Only'

.rc 2 |

:body

:h1.RAMADE - Enable use of RAM in segments A, D and E

:xmp

.rc 1 on

.rc 2 on

.sx c |Version 1.21    ||      Cornell Wright|

.sx c |1/12/87         ||   WRIGHT at PKEDVM8|

.rc 2 off

.rc 1 off

:exmp

:p.:hp3.WARNING - WARNING - WARNING - WARNING - WARNING:ehp3.

.rc 1 on

:ol

:li.RAMADE is a program which has the potential for conflicting with

the ROM and RAM addresses of existing and unannounced adapter cards.

:li.RAMADE has had only limited use and testing.  Please

read :hdref refid=prob. before using it.

:li.Use it with care and at your own risk.

:eol

.rc 1 off

:h2.Function

:p

This program enables the utilization of 192 k of RAM in

addition to the 640 k normal maximum for a PC. The

additional RAM must be installed at segment addresses A000,

D000 and E000. The 64k of RAM in segment A is contiguous

with the normal 640k and is used simply by extending the

memory size above 640. The 128k in segments D and E is not

contiguous with the rest of RAM.  It is used for loading

various resident extensions, thus freeing up RAM in the main

area (0-704k).

:p

See RAMCHIP QUERY on IBMPC for a discussion of methods for installing

additional memory.

.rc 1 on

:h2.Changes

:dl tsize=22

.rc 2 on

:dt.Ver 1.21 - 1/12/87

:dd.Added save restore of segment register around DOS call to invoke

secondary command processor.  This is to fix a problem under DOS 2.1.

:p.Added code to modify the DOS memory allocation blocks so that DOS

will "forget" about the memory above the contiguous area.

This fixes the problem where Filecmd will not cancel because it thinks

something is installed above it.

:p.Added test for 0 length clear in address table.

.rc 2 off

:dt.Ver 1.10 - 11/05/85

:dd.Placed constants for storage addresses in a table in the module

as an interim approach to configurability.  This will facilitate

patching the addresses.  See :hdref refid=zap. for more information.

:dt.Ver 1.00 - 11/03/85

:dd.First release.

:edl

.rc 1 off

:h2.Description

:p

The memory size is set to the end of segment E by resetting

the bios memory size and rebooting.  Storage in segments A,

D and E is tested and cleared. Miscompares are detected,

but no attempt is made to handle the more common case of

parity checks during the test.

:p

Resident extensions are loaded in segments D and E by a

secondary command processor which is invoked by RAMADE.

After issuing a SETBLOCK function call to free the 128 K

bytes in segments D and E, RAMADE invokes the secondary

command processor with a command of 'AUTOLOAD'.  This should

be a bat file which contains the commands to load the

resident extensions.

:p

After loading the resident extensions, RAMADE issues other

memory management function calls which leave segments B and

C allocated and free up unused memory below segment B.

:p

In addition, the warm boot flag in low storage is reset.

The system bios sets this flag when doing a ctl-alt-del

reset, but never clears it, apparently relying on power

dropping to clear the area.  (I've recently installed 256k

chips on my system board, and after this installation,

sometimes my system was warm booting when it should have

been cold booting after turning the power off briefly.)

:h2.Use

:ol

:li.Install 832 k of RAM in your PC as described above.

:li.Create AUTOLOAD.BAT.  This should contain the commands

to invoke the resident extensions you wish to load in segments

D and E. (See sample below.)

:li.Invoke RAMADE early in your AUTOEXEC.BAT.  (See sample

below.)

:eol

:h4.Sample AUTOEXEC.BAT:

:xmp

echo off

cls

verify on

cd \sys

path c:\cmd;c:\dos31;

ramade >nul

keyin >nul

pcutil sw cl

pcutil sw cl

copy graphic.fx prn >nul

xtime

echo �

echo �

iplmsg

prompt $e[H$e[0;34;43m$p$e[K$e[;56H$t$h$h$h   $d$e[0m$e[25;H$e[33m��#$e[36m

ver

cd \misc

keyin [4]

:exmp

:h4.Sample AUTOLOAD.BAT:

:xmp

echo off

cls

fastk /s /f /p >nul

*keyin >nul

retrieve retrieve.pro >nul

puff >nul

spi 32 >nul

print /d&gml.lpt1 >nul

fspause off /tm&gml.on /tv&gml.5 >nul

set ff=c:\global

filefac /i /o /nc /nm >nul

cls

scrfix.exe

pctermon.com >nul

fc3 /q

:exmp

:h4.MEMMAP output from the above:

:xmp

Seg 07E6  M  Size 04E4  Owner 0008

Seg 0CCB  M  Size 00C0  Owner 0CCB

Seg 0D8C  M  Size 000A  Owner 0CCB

Seg 0D97  M  Size 0004  Owner 0000

Seg 0D9C  M  Size 0005  Owner 0DA9

Seg 0DA2  M  Size 0006  Owner 0DA2

Seg 0DA9  M  Size 0158  Owner 0DA9

Seg 0F02  M  Size 000A  Owner 0F0D

Seg 0F0D  M  Size A0F2  Owner 0F0D

Seg B000  M  Size 2000  Owner 0DA2

Seg D001  M  Size 00CB  Owner 0000

Seg D0CD  M  Size 0005  Owner D0D3

Seg D0D3  M  Size 002F  Owner D0D3

Seg D103  M  Size 0005  Owner D109

Seg D109  M  Size 026D  Owner D109

Seg D377  M  Size 0005  Owner D37D

Seg D37D  M  Size 0561  Owner D37D

Seg D8DF  M  Size 0005  Owner D8E5

Seg D8E5  M  Size 0840  Owner D8E5

Seg E126  M  Size 0005  Owner E26C

Seg E12C  M  Size 013F  Owner E12C

Seg E26C  M  Size 0055  Owner E26C

Seg E2C2  M  Size 0006  Owner E2C9

Seg E2C9  M  Size 005E  Owner E2C9

Seg E328  M  Size 0006  Owner E32F

Seg E32F  M  Size 0011  Owner E32F

Seg E341  M  Size 0006  Owner E348

Seg E348  M  Size 0042  Owner E348

Seg E38B  M  Size 0006  Owner E392

Seg E392  M  Size 0850  Owner E392

Seg EBE3  Z  Size 041D  Owner 0000

:exmp

:h2 id=prob.Cautions, Problems and Notes

:p.This program is still somewhat experimental.  It has had limited

use and testing.  Use it with caution, and let me know if you have any

problems.

:h4.Testing Environment

:ul

:li.PC/XT with 832k RAM

:li.MicroScience 20 meg file and Western Digital controller

:li.DOS 3.1

:li.DOS extensions as listed in the sample installation

:eul

:h4.Known Problems

:ol

:li.When KEYIN is installed in the discontiguous area and is later

invoked from the lower area, I get a 'GENERAL FAILURE' on the C drive.

This could be related to my particular controller.  I now have KEYIN

installed in the lower area.

:li.Norton Utilities 'Unerase' function fails with 'Error 7'.  It appears

to update the FAT, but not the directory.  I used DR to get out of trouble

on this one.

:li.MASM was failing with 'OUT OF MEMORY'.  I applied a patch from PROGRAM

PATCHES on IBMPC and everything is fine.

:eol

.rc 1 on

:h2 id=zap.Patching RAMADE to use Different Addresses

:p

The RAMADE.COM module contains a table of segment addresses and lengths.

The existing values in the table support the addresses described in the rest

of this document.  If your PC has a different adapter configuration which

requires a different arrangement of RAM above 640 K, it may be possible

to patch this table to configure RAMADE to your PC.

:p

RAMADE views RAM as consisting of a contiguous area which starts at 0 and

a discontiguous area which starts above the end of the contiguous area (with

some intervening gap).  There are values in the table to identify these areas.

RAMADE also tests and clears 3 areas of RAM.  The starting addresses and

lengths of these areas are also in the table.

.rc 2 on

If the length is set to 0000, clearing that area will be skipped.

.rc 2 off

:p

.rc 2 on

The table is at offset 3C8h in the com module.  It is immediately proceeded

by the ASCII string 'ADDR TAB'.  The table contains 9 words:

.rc 2 off

:dl

:dt.+00

:dd.End of the contiguous area.  Paragraph address of the first byte after

the contiguous area.  Default is b000h.

:dt.+02

:dd.Start of the discontiguous area.  Paragraph address of the first byte of

the discontiguous area.  Default is d000h.

:dt.+04

:dd.End of the discontiguous area.  Paragraph address of the first byte after

the discontiguous area.  Default is f000h.

:dt.+06

:dd.Start of the first area to test and clear.  Paragraph address of the first

byte of the area.  Default is a000h.

:dt.+08

:dd.Length of the first area to test and clear.  Length in words.

Default is 8000h.

:dt.+0A

:dd.Start of the second area to test and clear.  Paragraph address of the first

byte of the area.  Default is d000h.

:dt.+0C

:dd.Length of the second area to test and clear.  Length in words.

Default is 8000h.

:dt.+0E

:dd.Start of the third area to test and clear.  Paragraph address of the first

byte of the area.  Default is e000h.

:dt.+10

:dd.Length of the third area to test and clear.  Length in words.

Default is 8000h.

:edl

:p

Remember that words are stored last-byte-first.  Thus b000 is stored as

00b0.  Got that ?

.rc 1 off

:egdoc