ATAINF Version 1.3 - This is the fifth public release of ATAINF.

http://www.ncf.carleton.ca/~aa571

ATAINF - What is it?

------------------

ATAINF is a program I wrote to find the parameters and capabilities

of ATA, ATAPI and SCSI drives.  ATA drives are more commonly known as

IDE or EIDE hard drives, and ATAPI are more commonly known as IDE

versions of CDROM drives.  There were only a couple of programs

out there that would dump some of the paramters of IDE hard drives,

but they had no support for CDROM and most of the time they did

not follow the ATA specs. ATAINF now supports SCSI drives

through the ASPI drivers, ATAPI drives through the ASPI drivers in

Windows 95, ATAPI drivers through the ATASPI drivers, and most other

drives through the native ATA/ATAPI interface.

A number of people were asking when I would support Windows 95.  As of

version (1.1), it now supports '95 when run in a DOS box.  There will not

be a GUI version of the program any time soon.  I have also added

support for SCSI CDROM drives.

One note for Windows 95 users:  At the moment there is no way to get

the information on your ATA drives when running in a DOS box. I'm

looking into the problem, and will release a new version if I can

get it working.  Microsoft seems to have made CDROM drives appear

as if they are SCSI drives, even if they are ATAPI drives when

running '95.  The result of this is that the drive is no longer

referred to by IO address, IRQ and device #, but is accessed by

Adapter #, SCSI ID, and LUN.  I have included a second program

called SCSIPING in this distribution which will scan all the

ASPI devices on your machine and should show the Adapter #, ID and

LUN of any drives.

Once I got part of ATAINF running I tried it on a number of machines

to make sure that it would run.  What I found was VERY interesting.

I tried running it on the new Creative Labs HexSpeed drive, and

I found that the drive thinks it is a 2x drive and not a 6x drive.

It displayed that the maximum throughput of the drive was 353 KB/s

instead of 1059 KB/s which is what a 6x drive should display.

I then tried running a performance benchmark program on the drive

and found I couldn't even get 2x throughput.  All I had to do was

to take the dump from ATAINF with me and I had no trouble returning

the drive to the store because of the crap.  I spent $494 CDN for

the drive and only got the performance of a $70CDN drive.  If I

didn't have ATAINF I would have been screwed.  The following table

will show you how to map the throughput numbers displayed by the

program to the speed of the drive.  You are going to find many drive

manufacturers using different numbers to describe their drives.  For

example you may find someone describing a 4x drive as 600KB/s.  This

would be fine if there were only 2048 bytes per sector on a CD, which

is the sector size of a plain data CD.  In reality a sector is 2352

bytes per sector.  On a data CD the extra bytes are sync marks, error

checks etc.  When you try and start using other modes with the drive,

like XA etc, then you will be limited to the 600 KB/s instead of the

actual 706 KB/s needed to read XA sectors at 4x speed.

Speed      Data Rate (KB/s)

---------------------------------

1x      |       176

2x      |       353

2.2x    |       387

3x      |       528

4x      |       706

6x      |      1059

8x      |      1412

10x     |      1765

16x     |      2824

A significant item of information that is returned by ATAINF is

whether the can read DA frames or not.  The following lines are

from my Sony drive which can read DA:

Drive does support reading Red Book audio using the Read-CD command

Drive does support reading Red Book audio with an accurate stream

This means that another of my programs CDDA will indeed work with

this drive.  I hear from a lot of people who are trying to use

CDDA and blame my program for the fact that it doesn't work.  By

using ATAINF, anyone with an ATAPI drive can tell if is the fault

of the drive, the fault of the driver or my fault that it doesn't

work.  For those of you that are interested CDDA is a program which

allows digitally perfect samples from audio CDs.  There is a pointer

to the most recent version of CDDA on my home page:

http://www.ncf.carleton.ca/~aa571

At this moment the most recent version is 1.6, but may take a while to

trickle down through the new Simtel system.

Another thing that you can tell with ATAINF is which mode your

IDE hard drive thinks it is running, as opposed to the maximum

mode it is capable.  I found that on my machine my drive could handle

up to mode 4, but it was stuck in mode 1.  It is just a problem with

my drivers, but I would have never known that I wasn't getting the

most out of my drive.

When running under a DOS window under Win95, some machines may support

using ATAPI drives through the ASPI interface.  However, bugs in

Microsoft's drivers prevent this from working on some machines. I have

worked a long time to get this working on a lot of machines, but I

can't guarantee it will work on all machines.  It has taken a significant

amount of effort to get around many of the problems in the device drivers

written by Microsoft.  It appears to be worth the trouble so that you

can have much more control over the drives.

To use the ASPI interface to the ATAPI drives in a DOS window under

Windows 95, you have to use the /95ATAPI option.

As of version 1.3, you can also use ATAINF with ATAPI drives through

the ATASPI drivers.  These drivers are for DOS and Win95 and usually

come as part of "PowerIDE" from Future Domain when you buy their

IDE controller.  However, as far as I can tell, the drivers will

work on any IDE controller, not just the ones from Future Domain.

Using the ATASPI drivers is probably the best way to use ATAINF and

ATAPI drives.  It means that I have the most control over the

drive and can bypass all the normal device drivers.  In this way

I can be sure that the device drivers are not at fault when ATAINF

doesn't work.

If you have a Future Domain IDE controller and PowerIDE, you are

all set.  You just have to add the device drivers in config.sys

and autoexec.bat. (see below my example)

If you don't have a Future Domain product, you will have to find

the ATASPI drivers elsewhere.  You might want to have a look

at Mediavision's FTP site (ftp.mediavis.com), or their web site

(www.mediavis.com), where they keep update driver files for

many of their products.  You might find update drivers for your

MV600, which runs on a Future Domain controller.

The two lines needed in my config.sys file are:

device=c:\ataspi\ataspi16.sys /c:170,15,376h /v

device=c:\ataspi\fdatacd.sys /d:mscd000

The line needed in my autoexec.bat file is:

c:\ataspi\mscdex /d:mscd000 /v /e

How do I use this program?

--------------------------

To run ATAINF, you can just run it from the command line and it

will do the searching for drives in your system.  Because of the

tendancy of the industry to rarely follow standards exactly, the

search routines in ATAINF may not necessarily find or identify

the drives in a machine.  I have also added command line options to

manually enter IO addresses, IRQ values and device numbers.  The

IO addresses can be 1f0, 170, 168, 1e0, or 1e8.  The IRQ values can

be 10, 11, 12, 13, 14, or 15.  The device number can be either 0 or 1.

When using the ASPI/ATASPI/95ATAPI options you can optionally enter

the Adapter number, SCSI ID number and the Logical Unit Number (LUN).

The following is a dump of the usage screen that is displayed by the

program.

-----------------------------------------------------------------------

Usage: ATAINF </ASPI> </ATASPI> </95ATAPI> </ATAPI> options

/ASPI    - Use ASPI drivers to talk to SCSI drive

/ATASPI  - Use ATASPI drivers to talk to ATAPI drive

/95ATAPI - Use Win 95 ASPI drivers to talk to ATAPI drive

/ATAPI   - Use native mode ATA/ATAPI drivers

Options:

IRQ    - Specify the IRQ of the IDE controller

DEVICE - Specify the Device number of the IDE drive

IO     - Specify the IO address of the IDE controller

ADAPTER- Specify the ASPI/ATASPI adapter number

ID     - Specify the ASPI/ATASPI device ID number

LUN    - Specify the ASPI device logical unit number

Examples:

DOS only ATA/ATAPI drive at IO=170, IRQ=15, Device#=0

atainf /io 170 /irq 15 /device 0

DOS only ATA/ATAPI drive at IO=170, IRQ=15

atainf /io 170 /irq 15

DOS only ATA/ATAPI drive at IO=170

atainf /io 170

DOS or Windows SCSI drive at Adapter=0, ID=2, LUN=0

atainf /aspi /adapter 0 /id 2 /lun 0

DOS or Windows SCSI drive: search all Adapters, all IDs, all LUNs

atainf /aspi

Windows '95 ATAPI drive at Adapter=0, ID=0

atainf /95atapi /adapter 0 /id 0

DOS ATASPI drivers with ATAPI drive at Adapter=0, ID=0

atainf /ataspi /adapter 0 /id 0

-----------------------------------------------------------------------

eg:

ATAINF /io 1f0 /irq 14 /device 0

(looks only at drive 0 at IO 1f0 and IRQ 14)

or

ATAINF /io 170 /irq 15

(looks at both drive 0 and 1 at IO 170 and IRQ 15)

The following is a dump from my machine which has a Conner 1.2GB hard

drive and a Toshiba 5302 4x CDROM drive:

------------------------------------

ATA_INF 1.3

Copyright (c) 1995/1996 - Computall Services.

Running DOS version 6.22

Will scan IO ports for drives

Will use default IRQs for IO ports for drives

Will scan device numbers for drives

Checking for controller at 0x1f0 ...

Found controller at 0x1f0

Checking for ATA drive at device 0 ...

Found ATA drive at device 0

BIOS Reports

Number of Cylinders               : 619

Number of Heads                   : 64

Number of Sectors per Track       : 63

ATA Identify Device parameters

Model Number                      : Conner Peripherals 1275MB - CFS1275A

Serial Number                     :           EVB8CT2

Hard Disk Reports

Number of Cylinders (Fixed)       : 2477

Number of Heads                   : 16

Number of Sectors per Track       : 63

Controller Revision Number        : 0.28

Able to do Double Word Transfer   : No

Controller type                   : a dual ported multi-sector buffer capable of

simultaneous with a read caching capability.

Controller buffer size (bytes)    : 0

Number of ECC bytes transferred   : 4

Peripheral Device Type Information:

Rotational speed tolerance is > 0.5%

Disk Transfer rate > 10MBps

Fixed Drive

Head switch time >15 usec

Not MFM encoded

Hard sectored

# of secs/interrupt               : 16  Current setting 16

Able to do Double Word Transfer   : No

LBA support                       : Yes

1219.2MB of LBA addressable              1219.1MB in CHS mode

DMA support                       : Yes

IORDY supported                   : Yes

IORDY can be disabled             : Yes

PIO data txfer cycle timing mode  : 1

SW DMA txfer cycle timing modes   : none, Active: none

MW DMA txfer cycle timing modes   : none, Active: 0

ATA-2 features

Advanced PIO txfer modes supported           : 4 3

Min MW DMA txfer cycle time/word             : 120 ns   16.7MB/s

Mfg Recommended MW DMA txfer Cycle Time      : 120 ns   16.7MB/s

Min PIO txfer Cycle Time w/o Flow Control    : 270 ns   7.4MB/s

Min PIO txfer Cycle Time w IORDY Flow Control: 120 ns   16.7MB/s

Checking for ATA drive at device 1 ...

No ATA drive found at device: 1

No Data Request: timeout

Checking for ATAPI drive at device 1 ...

No ATAPI drive found at device 1

No Data Request: timeout

ata_status = 0x00

ata_error = 0x00

irq_source = 0x00

Checking for controller at 0x170 ...

Found controller at 0x170

Checking for ATA drive at device 0 ...

No ATA drive found at device: 0

No Data Request: timeout

Checking for ATAPI drive at device 0 ...

Found ATAPI drive at device 0

ATAPI Identify Device parameters

Device Type                       : CD-ROM Device

Removable                         : Yes

CMD DRQ Type                      : Interrupt DRQ

Command Packet Size               : 12 bytes

Serial Number                     : 1100302955

Firmware Revision Number          : 1095

Model Number                      : TOSHIBA CD-ROM XM-5302TA

DMA support                       : No

LBA support                       : Yes

IORDY supported                   : Yes

IORDY can be disabled             : No

Overlap Operation support         : No

Proxy Interrupt support           : No

Interleaved DMA support           : Yes

PIO data xfer cycle timing mode   : 0

SW DMA txfer cycle timing modes   : none, Active: none

MW DMA txfer cycle timing modes   : none, Active: none

Typical release time after command: 0ns

Typical release time after service: 0ns

Major version number              : Not supported

ATAPI Drive Capabilities and Mechanical Status parameters

(4a)Warning: No Interrupt Timeout: will continue anyway

ATA buffer count returning: 56 bytes

Error recovery parameters: 0

Error retry count: 5

Drive Inactivity timer: 32 seconds

Number of MSF-S Units per MSF-M Unit: 60

Number of MSF-F Units per MSF-S Unit: 75

Logical Block Per Second of Audio Playback: 0

CDDA Output Port 0 Channel Selection: Channel 0, Volume: 0xff

CDDA Output Port 1 Channel Selection: Channel 1, Volume: 0xff

CDDA Output Port 2 Channel Selection: Muted, Volume: 0x00

CDDA Output Port 3 Channel Selection: Muted, Volume: 0x00

Maximum Speed (kB/s): 706

Number of Volume Levels Supported: 255

Buffer size supported by drive (kB): 256

Current selected speed (kB/s): 706

Drive does support Audio Play

Drive does not support delivering a composite Audio/Video stream

Drive does not support digital output (IEC958) on port 1

Drive does not support digital output (IEC958) on port 2

Drive does support reading sectors in Mode 2 Form 1 (XA) format

Drive does support reading sectors in Mode 2 Form 2 format

Drive does support reading multiple session or Photo-CD discs

Drive does support reading Red Book audio using the Read-CD command

Drive does not support reading Red Book audio with an accurate stream

Drive does support commands that return sub-channel data

Drive does not support de-interleaving R-W sub-channel data

Drive does not support C2 error pointers

Drive does support returning the ISRC information

Drive does support returning the UPC information

Drive does support locking media in drive

Drive is in allow lock state

Drive's prevent jumper is present

Drive does support the eject command

Drive loading mechanism is Tray type loading mechanism

Drive does support separate audio level controls

Drive does support separate audio mute controls

Drive does not support Disk Is Present information

Checking for ATA drive at device 1 ...

No ATA drive found at device: 1

No Data Request: timeout

Checking for ATAPI drive at device 1 ...

No ATAPI drive found at device 1

No Data Request: timeout

Checking for controller at 0x1e0 ...

No controller found at 0x1e0

Checking for controller at 0x1e8 ...

No controller found at 0x1e8

Checking for controller at 0x168 ...

No controller found at 0x168

-------------------------------------------------------

Copying and Registration (does he mean this costs $$$?)

-------------------------------------------------------

I am allowing full freedom to copy this program.

It IS a fully functional version.

It is NOT Crippleware!

There is NOT a pro version available for an unreasonable sum of money.

If you don't agree with the shareware policy, then you can:

a) use the program anyway and violate the copyright, and as such

you will probably end up rotting in HE double hockey stick

b) don't use the program at all

The registration fee for using ATAINF is now $15 US.  I believe that

this is a fair amount given the amount that the program does.  This

amount can easily be covered by the amount you can save by finding

out that you are being sold a piece of crap before you buy it.

I won't be asking for a fee every time a new version comes out.  This

fee will give you a lifetime (as long as I bug fix and add features)

of free updates via the Internet.  I will also notify registered users

by email of any new versions for a year.

My mail address is:

Jim McLaughlin

337 Nautica Ct.

Indian Harbour Beach, FL

USA

32937

If you have any questions, my email address is jmclaugh@nortel.ca, but RTFM

before you start sending me then questions.

*******

NOTE: the email address change from version 0.9a

NOTE: the address address change from version 1.1a

*******

I would also like to try and keep all the files together and unchanged

in the archive, so if you are passing this around don't change the files.

In this distribution the files contained are:

ATAINF.EXE - the program itself

ATAINF09.EXE - the first version of the program itself

ATAINF.DOC - this document file

SCSIPING.EXE - a program to scan for drives when using ASPI drivers

ATAINF09.EXE is included because there are some machine/drive combinations

that just won't work with ATAINF 1.0 and later.  I don't know why, but

they sometimes work with the first version of ATAINF.  I am only including

it for the people who can't get the newer versions to work.

If you are one of those Shareware houses, I don't want to see this program

available for $6.99 plus shipping and handling.  I don't like seeing

the average Joe getting burned for a 10 cent disk, 1 cent label and 2 cents

worth of labour to make the disk, especially when he can get 600+ Meg

of really good stuff from something like the Simtel CD for $25

(Thanks to Keith for keeping Simtel running).

How to get hold of me for help.

-------------------------------

If you want to make suggestions or want to send compliments you can contact

me at the address below.  I might even listen to a complaint or two.

If you would like assistance in solving problems, please include a

dump from the.  You should be able to run the program like this and

get the dump in a text file:

ATAINF /atapi /io 170 /irq 15 /device 0 >dumpfile.txt

I have real difficulty in tracking down problems without the

dumps.  In the past the majority of people asking for help just send

along a short note telling me my program doesn't work, and what

they remember the error messages saying.  This just doesn't cut it.

I have to have the dumps to help.

The bottom line - RTFM before writing me.  Also read the alt.cd-rom

FAQ.  Many of your questions can be answered by reading.

EMAIL: jmclaugh@nortel.ca

Changes in version 1.3 from  1.0a

---------------------------------

- added full support for ATAPI drives through either ASPI under Win95

or ATASPI under DOS

- cleaned up the drive search routines

- cleaned up the displayed info

- removed the dumps from all the cdrom and hard drives - there is a much

better repository at the Tardis UK web site - there are pointers to

it on my web site - check it out.

Changes in version 1.1a from 1.0a

---------------------------------

- made significant changes to the command line options to add

ASPI and SCSI

- added support for SCSI drives through the ASPI interface

- added support for ATAPI drives through the ASPI interface on

machines running Windows 95

- added more detail and information in the reports generated

- fixed a problem that caused a couple of different drives (ones

that weren't very ATAPI compliant) to only send half of the

data to the computer and then hang

Changes in version 1.0a from 0.9a

---------------------------------

- please make note that my new email address is jmclaugh@nortel.ca

- added a check to see if the program is running is a DOS box under Windows

this was done because bad things sometimes happen when you run ATAINF

and you are not in DOS

- the most recent version of ATAINF can be found by looking at my new

homepage for pointers: http://www.ncf.carleton.ca/~aa571

this page contains pointers to the most recent drivers for most

every cdrom drive made

- a number of bugs have been fixed

- ATAINF will display even more information about your cdrom drive than

before

- there is a work around for the problem in that many drives say they

use interrupts, but none are ever generated

- included dumps from many cdrom and hard drives  - please send me the

dumps so I can build a complete list - many people will find this

information useful when trying to decide on a new drive

- added the IO address of 0x1E0 and the interrupts 12 and 13

Known bugs

----------

- there are no known bugs at this time

- the inability to get ATA drive information when running '95 is

not a bug, but is something that only seems to run in plain

DOS, and not in a DOS box.  I'm working on it.