SVGA Accelerator Software Drivers-1

GUIDE TO SVGA ACCELERATOR SOFTWARE DRIVERS

----------------------------------------------------------------

Your SVGA Accelerator video hardware is packaged with software

drivers for running several popular MS-DOS-based applications in

the Super VGA modes. The drivers and installation program

(INSTALL.BAT) for MS-DOS-based applications are on the MS-DOS

Application Drivers Diskette.

Press [Page Up] and [Page Down] to view this document a screen at

a time. Press the up and down direction keys to scroll through

the document. To exit the document at any time, press [Esc].

NOTE: The drivers are supplied by a third-party vendor for the

convenience of advanced users.

Document Conventions

-----------------------------------------------------------------

Throughout this document, the following conventions are used to

differentiate various types of information:

.    Words enclosed in square brackets ([ ]) represent keys on

the keyboard. For example: [Enter].

.    Groups of keys are printed like this: [Ctrl + Alt + Del].

Press and hold the keys in the order shown ([Ctrl], [Alt],

and [Del], in this example). Then, release the keys.

.    Words enclosed in angle brackets (<>) are placeholders for

variable information.

.    Filenames, directory names, command names, and command

switches are shown in all uppercase. For example, C:\WP and

WDPVGA1A.WPD. You can enter such information in any mixture

of upper- and lowercase characters.

SVGA Accelerator Software Drivers-2

CONTENTS

-----------------------------------------------------------------

INTRODUCTION .................................................. 3

MS-DOS Application Drivers Diskette ...................... 3

Before You Use the Software Drivers ...................... 4

Super VGA Notes .......................................... 5

Driver Installation for MS-DOS Applications .............. 6

VGAMODE.EXE .............................................. 7

AUTOCAD 386, AUTOSHADE 386, RENDERMAN, AND 3D STUDIO .......... 8

Configuring the Applications ............................. 9

AUTOCAD ...................................................... 10

Configuring AutoCAD Releases 9 and 10 ................... 11

Running AutoCAD Releases 9 and 10 ....................... 12

Changing the Default AutoCAD Colors

for 256-Color Drivers ................................... 14

Format for RGB.256 ...................................... 15

Changing the Default AutoCAD Colors

for 16-Color Drivers (Versions 1.11 and Above) .......... 16

Format for RGB.16 ....................................... 16

AUTOSHADE .................................................... 18

CADVANCE ..................................................... 19

GENERIC CADD ................................................. 19

LOTUS 1-2-3 AND LOTUS SYMPHONY ............................... 21

PCAD ......................................................... 22

VERSACAD ..................................................... 23

WORDPERFECT .................................................. 25

Running WordPerfect 5.0 and 5.1 with Graphics Screen

Driver .................................................. 25

Running WordPerfect 4.2 in 132-Column Text Modes ........ 26

WORD ......................................................... 27

Selecting the Video Driver for Word 5.0 and Word 5.5 .... 27

USING VESA.EXE ............................................... 28

VGA AND SUPER VGA PROGRAMMING ................................ 29

Standard VGA Modes ...................................... 30

Super VGA Modes ......................................... 31

Programming Considerations .............................. 32

SVGA Accelerator Software Drivers-3

INTRODUCTION

-----------------------------------------------------------------

The following tables summarize the major files on the driver

diskette for SVGA Accelerator video. Refer to these tables to

determine which drivers are available for the applications you

are running.

MS-DOS Application Drivers Diskette

-----------------------------------------------------------------

Program          Description

INSTALL.BAT      Command to run installation

README.BAT       Command to display the README file

PRINTME.BAT      Command to print the README file

VGAMODE.EXE      VGA mode-switching program

LOOK.COM         Program used to display the README

file

README.TXT       Text file containing documentation

on video software drivers

ACAD.LIF         AutoCAD Releases 9, 10, 11,AUTOCAD

386 products, and AutoShade 1.x

CADVANCE.DRV     Cadvance 1.2-3.0

GENERIC.LIF      Generic CADD Level 3

LOTUS.LIF        Lotus 1-2-3 Releases 2.0, 2.01, 2.2

and Symphony Releases 1.1, 1.2, 2.0

PCAD.LIF         PCAD Level 2

UTILITY.LIF      PROGRAM.TXT and VGAMODE.EXE

VCAD.LIF         VersaCAD 5.3, 5.4, and 5.4/386

WP.LIF           WordPerfect 5.0 and 5.1

WORD.LIF         Microsoft Word 5.0 and 5.5

!TDY8104.CFG     EISA configuration file

SVGA Accelerator Software Drivers-4

SVGA Accelerator video supports the following Super VGA modes in

addition to the standard VGA modes:

.    320x200 graphics

.    1024x768 graphics

.    800x600 graphics

.    1280x960 graphics

.    1280x1024 graphics

.    640x400 graphics

.    640x480 graphics

.    80-column text in 25, 34, 43, or 50-line text modes

.    132-column text in 25, 28, 44, or 50-line text modes

Refer to "Super VGA Notes" for more information.

The following sections provide instructions for using several

popular applications with the Super VGA modes.

Before You Use the Software Drivers

-----------------------------------------------------------------

.    Use the MS-DOS DISKCOPY command to make working copies of

the drivers diskette. Use only the working copies to install

the drivers.

.    Do not write protect the working diskette. The INSTALL

program will write to them when installing drivers.

.    Be sure that your SVGA video is correctly configured.

(If the video controller is an adapter, refer to the adapter

documentation for instructions.)

.    Have the command processor (COMMAND.COM) in the path, or

copy it to the current directory.

.    Run INSTALL from the working diskette you created. (You

cannot run INSTALL from your hard disk because INSTALL does

not let you install to the source drive.) Type INSTALL and

press [Enter] on the command line to run INSTALL.

.    If the installation process fails, consult your application

documentation to complete the installation.

.    Remember to install the DOS-based software applications

before you install the application specific drivers.

SVGA Accelerator Software Drivers-5

Super VGA Notes

------------------------------------------------------------

Following is a summary of the Super VGA modes:

1280 and 1024 Resolution Graphics

non-interlaced 1024x768 graphics in 16 or 256 colors from

a palette of more than 262,000 colors.

non-interlaced 1280x960 and 1280x1024 graphics in 16 colors.

These modes require a multiple-frequency monitor.

800x600 Graphics

The 800x600 graphics mode enables the screen to display at least

56% more information than standard VGA modes. You might need to

use the vertical and horizontal size and position controls on

your monitor to display the entire 800x600 graphics mode image

without distortion.

These modes require a multiple-frequency monitor.

256-Color, 640x400 and 640x480 Graphics

The 256-color, 640x480 graphics mode provides five times the

resolution of standard VGA 256-color graphics.

The 256-color, 640x400 graphics mode provides four times the

resolution of standard VGA 256-color graphics.

132-Column Text

The 132-column text mode is available in 25, 28, 44, and 50-line

formats. These Super VGA text modes enable you to view text in

this format only with applications that support these modes.

SVGA Accelerator Software Drivers-6

Driver Installation for MS-DOS Applications

-----------------------------------------------------------------

Install all MS-DOS applications before running INSTALL to install

the video drivers for the applications. If you are prompted to

choose a video standard during installation, select VGA video.

You can use the VGAMODE program to select the video mode or

emulate specific video standards as directed in the application

instructions.

To install a Super VGA video driver for an MS-DOS application:

1.   Insert the MS-DOS Application Drivers Diskette into a drive

and type:

<drive>:

Then, press [Enter] to access the diskette.

2.   Type:

install

and press [Enter] to display the main menu of the

drivers available on the diskette.

3.   Use the direction keys to scroll and highlight the

application driver you wish to install. Press [Enter].

4.   Follow the directions on the screen to install the driver.

INSTALL returns to the main menu when driver installation is

complete.

5.   Select the other application drivers you wish to install, or

exit the main menu to return to the operating system prompt.

This document also contains specific information for loading and

configuring drivers for AutoCAD 386, AutoCAD, AutoShade,

Cadvance, Generic CADD, Lotus 1-2-3, Lotus Symphony, PCAD,

VersaCAD, WordPerfect, and Microsoft Word. Refer to the

appropriate section for additional information.

To install the VGAMODE utility on your hard disk, run the INSTALL

program as described in the instructions for installing an MS-DOS

application driver. At the INSTALL main menu, select "Utilities."

Then select "VGAMODE.EXE." Follow the instructions on the screen

to install VGAMODE.

SVGA Accelerator Software Drivers-7

VGAMODE.EXE

-----------------------------------------------------------------

The VGAMODE.EXE utility enables your SVGA Accelerator to emulate

specific video standards. Refer to the video information in your

computer user's guide or adapter installation guide for more

information.

NOTES:

.    The drivers described in this documentation use a color VGA

configuration. If you are using a monochrome VGA monitor,

use the command VGAMODE COLOR to switch from monochrome VGA

mapping to color mapping before using the drivers. For more

information on monochrome and color mapping, refer to the

video information in your computer user's guide or adapter

installation guide.

.    The video modes supported by VGAMODE require both hardware

and software support. Check your software documentation and

choose only those modes that your application supports.

Remember that some modes may require a specific type of

monitor, such as non-interlaced or multiple-frequency.

SVGA Accelerator Software Drivers-8

AUTOCAD 386, AUTOSHADE 386, RENDERMAN, AND 3D STUDIO

-----------------------------------------------------------------

Use the following video drivers with Autodesk 386 products (with

DOS Extender). The drivers support AutoShade, 386, RenderMan, and

3D Studio, and AutoCAD 386 Release 11.

Autodesk Application Driver              Resolution and Colors

===============================================================

Non-Display List Drivers

AutoCAD 386          11PSHV18.EXP        1024x768 256 colors

AutoCAD 386          11PSHV88.EXP        800x600 256 colors

AutoCAD 386          11PSHV68.EXP        640x480 256 colors

Display List Drivers

AutoCAD 386          11PDLV14.EXP        1024x768 16 colors

AutoCAD 386          11PDLV84.EXP        800x600 16 colors

Display List Drivers

AutoCAD 386          11PDLV18.EXP        1024x768 256 colors

AutoCAD 386          11PDLV88.EXP        800x600 256 colors

SVGA Accelerator Software Drivers-9

Configuring the Applications

-----------------------------------------------------------------

To run the drivers for AUTOCAD 386, RenderMan, and 3D Studio,

follow these steps:

1.   Set the MS-DOS environment variable(s) to the pathname of

the driver you selected. Use the following format:

set dspadi=<drive>:\<driver path>\<driver filename>

where <drive> is the drive that contains the driver file,

<driver path> is the directory path of the application

driver, and <driver filename> is the filename of the driver

(as listed in the previous table) for the application you

want to configure.

2.   Type:

acad

to load AutoCAD 386. If you have not already done so,

configure the video display for AutoCAD 386 by choosing the

option P386 ADI V4.0/4.1 Display.

If you later need to change the driver configuration, access the

AutoCAD main menu and select "Configure AutoCAD" (number 5).

Then, select "Configure Video Display" (number 3) from the

configuration menu. A screen of configuration options is

displayed.

Press [Enter] to select an option. Some parameters, such a

"Single Screen," change from false to true when you press

[Enter]. When you press [Enter] for options with numeric values,

a window is displayed to illustrate your color choice.

SVGA Accelerator Software Drivers-10

AUTOCAD

-----------------------------------------------------------

The following table lists the Super VGA modes for which AutoCAD

Releases 9 and 10 can be configured:

Driver Filename    Resolution or Columns x Rows Number of Colors

================================================================

AutoCAD Releases 9 and 10

Non-Display List Drivers

DS800X16.EXE       800x600                      16

DS800FF.EXE        800x600                      256

DS102416.EXE       1024x768                     16

DS1K256.EXE        1024x768                     256

AutoCAD Release 10 Display List Drivers

WDVGADL1.EXE       1024x768                     16

WDVGADL8.EXE       800x600                      16

The MS-DOS Application Drivers Diskette contains display and

non-display list drivers for AutoCAD 10, and non-display list

drivers for AutoCAD 9. To use display list drivers, you must have

created expanded memory on your computer.

SVGA Accelerator Software Drivers-11

The drivers supplied for AutoCAD Release 9/10 enables you to use

Super VGA graphics capabilities of SVGA Accelerator video with

AutoCAD's Advanced User Interface.  The driver is in the form of

an AutoCAD Device Interface, or ADI, driver. ADI is a generalized

driver format that enables you to load drivers into memory before

running AutoCAD. To use a driver, you must have AutoCAD Release 9

with a Version 3.1 ADI "generic" driver or Release 10 with a

Version 4.0 ADI "generic" driver. The drivers conform to the

AutoCAD display device interface specification.

The drivers require that you use a multiple-frequency monitor

capable of displaying 800x600 or 1024x768 resolutions. If you use

a fixed-frequency VGA monitor, install AutoCAD Release 9/10 for

standard 640x480 VGA resolution.

Configuring AutoCAD Releases 9 and 10

------------------------------------------------------------

1.   Run INSTALL.BAT to copy the driver file from the MS-DOS

Application Drivers Diskette to your AutoCAD directory.

2.   Run the driver file. You will get a message that reads:

--- AutoCAD Device Interface (ADI) Display Driver ---

This indicates that the AutoCAD driver has been loaded into

memory.

3.   If you use a mouse or graphics tablet that requires a

separate driver, be sure that its driver is loaded at this

point.

4.   Execute ACAD to start the AutoCAD Release 9/10 program.

5.   From the main menu, choose Option 5 to configure AutoCAD

Release 9/10.

6.   From the configuration menu, choose Option 3, "Configure

video display."

SVGA Accelerator Software Drivers-12

7.   Choose the "ADI display v3.1 / "ADI display v4.0" driver

from the list of available video drivers.  If this option is

not available, be sure that you are using AutoCAD Release

9/10. You might need to obtain an ADI generic driver update

from AutoDesk or your AutoCAD dealer.  When asked for the

hexadecimal interrupt code, choose the default "7A" unless

"7A" causes a conflict with other software. See "Running

AutoCAD Releases 9 and 10" for information about changing

the interrupt vector.

8.   Be sure all other configuration options are set as

appropriate for your computer.

9.   Exit to the main menu, and save your configuration changes.

AutoCAD is now ready to run in the selected graphics mode.

Running AutoCAD Releases 9 and 10

-----------------------------------------------------------------

Configure AutoCAD as described above. Then, follow these steps:

1.   Run the driver from the AutoCAD directory.

For a non-display list driver, type:

<driver filename>

and press [Enter]. Do not include the extension in the

filename.

For a display list driver, type:

<driver filename> f<KB>

where <KB> is the number of kilobytes of expanded memory

that the display list driver should leave free for use by

AutoCAD. For example, to run WDVGADL1.EXE and leave 512KB

free, type:

wdvgadL1 f512

SVGA Accelerator Software Drivers-13

Options:

.    You can run AutoCAD with an inverted black background rather

than the standard white background by invoking the driver

with the "i" parameter. For example, type:

DS800X16 i

and press [Enter].

.    You might need to change AutoCAD vector interrupt code to

something other than the default "7A". Type:

DS800x16 <xx>

where <xx> is the two-digit hexadecimal vector interrupt

code.

2.   If you use a mouse or graphics tablet that requires a

separate driver, be sure that its driver is loaded at this

point.

3.   Execute the AutoCAD program.

The commands can easily be incorporated into a batch file such as

this:

DS800X16       (Loads video driver.)

MOUSE          (Loads mouse driver. Exact command depends on

type of pointer or mouse used.)

ACAD           (Starts AutoCAD program.)

SVGA Accelerator Software Drivers-14

Changing the Default AutoCAD Colors

for 256-Color Drivers

-----------------------------------------------------------------

The SETC 256 program enables you to change the 256 default

AutoCAD colors for two video modes:

.    Mode 5C (256-color, 800x600)

.    Mode 60 (256-color, 1024x768)

This program displays the 16 default colors and lets you change

the RGB (red, green, and blue) values for each color. When you

finish defining the desired colors, press [Enter] to save the RGB

values to a file called RGB.256.

Press [Insert] to select colors above Index 15. The color is

displayed in the upper left portion of the screen. Use the left

and right arrow keys to select the index. The corresponding color

is displayed. When the color for that index is correct, press

[Enter]. This marks the color to be written to the RGB.256 file.

When you mark a color, the word "written" is displayed and the

arrow moves to index 15. To view colors without making changes,

press [Esc] after selecting the index.

After you define all the colors, press [Enter] to save the RGB

values for indices 0 through 14.

Run SETC 256 from the AutoCAD directory. The driver looks for

RGB.256 there when you start AutoCAD. If the driver cannot find

RGB.256, AutoCAD uses the default colors.

SVGA Accelerator Software Drivers-15

Format for RGB.256

-----------------------------------------------------------------

RGB.256 is a text file with 16 to 256 lines. Each line contains

four decimal values and ends with a carriage return. The

following table illustrates the first 64 default values.

Index   R    G   B             Index    R  G  B

(0-225) Intensity (0-63)       (0-255)  Intensity (0-63)

0       0    0   0             32       50 24 0

1       63   0   0             33       50 37 25

2       63   62  0             34       40 20 0

3       0    63  0             35       40 30 20

4       0    63  63            36       31 15 0

5       0    0   63            37       31 23 15

6       62   0   63            38       28 13 0

7       63   63  63            39       28 21 14

8       23   23  23            40       63 47 0

9       31   0   0             41       63 55 31

10      63   47  31            42       50 37 0

11      0    31  0             43       50 44 25

12      0    40  40            44       40 30 0

13      20   30  40            45       40 35 20

14      37   25  50            46       31 23 0

15      48   48  48            47       31 27 15

16      31   0   0             48       28 21 0

17      31   15  15            49       28 24 14

18      28   0   0             50       63 62 0

19      28   14  14            51       63 62 31

20      63   14  0             52       50 49 0

21      63   38  31            53       50 50 25

22      50   11  0             54       40 40 0

23      50   31  25            55       40 40 20

24      40   9   0             56       31 30 0

25      40   25  20            57       31 31 15

26      31   7   0             58       28 27 0

27      31   19  15            59       28 27 14

28      28   6   0             60       47 63 0

29      28   17  14            61       55 63 31

30      63   31  0             62       38 50 0

31      63   47  31            63       44 50 25

SVGA Accelerator Software Drivers-16

Values for Indices 64 to 255 can be found by running AutoCAD and

then SETC 256.

Changing the Default AutoCAD Colors

for 16-Color Drivers (Versions 1.11 and Above)

-----------------------------------------------------------------

The SETC 16 program enables you to change the 16 default AutoCAD

colors. The program displays the default colors and lets you

change them by altering their RGB values. After you define all

the colors, press [Enter] to save the RGB values in the RGB.16

file.

Run SETC 16 from the AutoCAD directory. The driver looks for

RGB.16 there when you start AutoCAD. If the driver cannot find

RGB.16, AutoCAD uses the default colors.

Format for RGB.16

-----------------------------------------------------------------

RGB.16 is a text file with 16 lines. Each line contains five

characters (three digits and two spaces). The digits (0, 1, 2,

and 3) represent color intensities. The first digit is the red

intensity, the second is the green intensity, and the third is

the blue intensity.

Each line represents a color index. The first line represents

Index 0. The last line represents Index 15. Each line, including

the last, ends with a carriage return.

SVGA Accelerator Software Drivers-17

The default values are:

R G B

=====

0 0 0

3 0 1

3 3 0

1 3 1

0 3 3

1 1 2

3 0 3

3 3 3

1 1 1

3 0 0

2 1 0

0 2 0

1 2 2

0 0 3

2 0 2

2 2 2

SVGA Accelerator Software Drivers-18

AUTOSHADE

-----------------------------------------------------------------

AutoShade Version 1.x can be configured for use with any of the

following Super VGA modes:

Driver Filename Resolution or Columns x Rows Number of Colors

==============================================================

RD400.EXE       640x400                      256

RD480.EXE       640x480                      256

RD800.EXE       800x600                      256

RD1024.EXE      1024x768                     256

Upon installation, AutoShade creates a configuration file called

SHADE.CFG. To configure AutoShade to use the Super VGA modes:

1.   Delete the configuration file SHADE.CFG if you have

previously installed AutoShade.

2.   Copy the ADI rendering driver by running INSTALL.BAT from

the MS-DOS Application Drivers Diskette.

3.   Run AutoShade by typing:

shade

and pressing [Enter].

4.   Respond to the following prompts as indicated:

Prompt                Selection

===============================================

Select Display        IBM Video Graphics Array

Device

Select Rendering      Autodesk Device Interface

Display Device        Rendering Driver

Shade a Single        Yes

Screen (DEFAULT No)

Does FLIPSCREEN       Yes

require a redraw

(DEFAULT No)

SVGA Accelerator Software Drivers-19

CADVANCE

------------------------------------------------------------

Cadvance can be configured for use with the following Super VGA

modes:

Driver Filename Resolution or Columns x Rows Number of Colors

=============================================================

GS102416.DRV    1024X768                     16

GS800.DRV       800x600                      16

GSVGA.DRV       640x480                      16

To install Cadvance for use with one of these modes, run

INSTALL.BAT to copy the appropriate driver file from the MS-DOS

Application Drivers Diskette to the Cadvance directory. The next

time you run Cadvance, it uses the new resolution.

GENERIC CADD

------------------------------------------------------------

Generic CADD Level 3 can be configured for use with any of the

following graphics drivers:

Driver           Resolution or       Number of Colors

Filename         Columns X Rows

==========================================================

P1024XFF.VGD     1024x768            256

P1024X16.VGD     1024x768            16

P800X600.VGD     800x600             256

PARA800.VGD      800x600             16

P640X480.VGD     640x480             256

P640X400.VGD     640x400             256

SVGA Accelerator Software Drivers-20

To install a Generic CADD graphics driver, run the INSTALL.BAT

program on the MS-DOS Application Drivers Diskette. INSTALL

copies the selected driver to the directory containing Generic

CADD.

If INSTALL can find CONFIG.EXE, it runs this program so that you

can configure Generic CADD for use with the selected driver.

If INSTALL cannot find CONFIG.EXE, you must run CONFIG.EXE

manually from the system prompt.

To configure Generic CADD for use with a particular driver:

1.   When CONFIG.EXE displays the current configuration, press

any key as prompted.

2.   Select "1" to choose a Video Graphics Display.

3.   Choose the appropriate graphics driver.

4.   Select "Exit to DOS."

5.   Respond "Y" to save the selected driver.

You have completed the installation of Generic CADD Level 3 for

the appropriate Super VGA mode. You can start Generic CADD Level

3 in the normal manner, and it will use the selected video mode.

SVGA Accelerator Software Drivers-21

LOTUS 1-2-3 AND LOTUS SYMPHONY

-----------------------------------------------------------------

Lotus 1-2-3 (Releases 2.0, 2.01 and 2.2) and Lotus Symphony

(Releases 1.1, 1.2, and 2.0) can be configured for use with any

of the following drivers:

Driver Filename   Resolution or Columns x Rows Number of Colors

===============================================================

SIB2VGA.DRV for   1024x768                     16

8x9 font

SIB3VGA.DRV for   1024X768                     16

9x13 font

SP1_600S.DRV      800x600                      16

SP1_480S.DRV      640x480                      16

SPO_43S.DRV       132x43                       16

SPO_25S.DRV       132x25                       16

CAUTION: When you add new drivers to the 1-2-3 or Symphony

installation program, a file called SINGLE.LBR is created. If you

are using Lotus 1-2-3 Release 2.0 or Symphony Release 1.1, be

sure that neither the Utilities/Install disk nor the Lotus

subdirectory already contains a file called SINGLE.LBR. If such a

file does exist, delete it before running INSTALL.BAT to add the

new drivers. If you are using 1-2-3 Release 2.1 or Symphony 1.2,

you do not need to delete the SINGLE.LBR file.

Follow this procedure to install drivers for Lotus:

1.   Use the INSTALL.BAT utility to copy the drivers from the

MS-DOS Application Drivers Diskette to the 1-2-3 directory.

2.   Refer to your Lotus manual for installation procedures for

the drivers.

When using Lotus's INSTALL.EXE file, choose "Add New Drivers

to Library" from the ADVANCED OPTIONS menu to add the .DRV

files to the SINGLE.LBR files.

Then, choose "Modify Current Drivers Set" from the ADVANCED

OPTIONS menu to list the new drivers on the menu.

SVGA Accelerator Software Drivers-22

PCAD

------------------------------------------------------------

PCAD can be configured for use with either of the following

drivers:

Driver             Resolution or      Number of

Filename           Columns x Rows     Colors

=====================================================

DPVGA1K.DRV        1024x768           16

DWDPVGA.DRV        800x600            16

To install PCAD for use with 1024x768 or 800x600 graphics:

1.   Run INSTALL.BAT to copy the selected driver to the PCAD

directory.

2.   Change the DISPLAY line in the file PCADDRV.SYS, located in

the root directory, to reflect the change in the driver. For

example, use:

Display \pcad\tkcards\dwdpvga.drv

for the 800x600 driver.

Use:

Display \pcad\tkcards\dpvga1k.drv

for the 1024x768 driver.

SVGA Accelerator Software Drivers-23

VERSACAD

------------------------------------------------------------

VersaCAD DESIGN Versions 5.3, 5.4, and 5.4/386 can be configured

for use with either of the following drivers:

Driver Filename   Resolution or Columns x Rows Number of Colors

=====================================================

VerscaCAD 5.4/386

PVGAP.EXE         1024x768                     16

PVGA256P.EXE      800x600                      256

640x480                      256

640x400                      256

VersaCAD 5.4

PVGA1024.EXE      1024x768                     16

PVGAFF.EXE        800x600                      256

640x480                      256

640x400                      256

VersaCAD 5.3

PVGA1024.EXE      1024x768                     16

To configure VersaCAD 5.3, 5.4, or 5.4/386 for use with the Super

VGA drivers:

1.   Run INSTALL.BAT to copy the driver.

2.   Run ENVIRO.EXE to set up the new screen driver.

3.   Use Environment Menu 3 to select the 1024x768 or 800x600

driver.

SVGA Accelerator Software Drivers-24

4.   Depending on the VersaCAD version and the desired

resolution, start VersaCAD with one of the following

commands:

VCAD53 1024

for 16-color, 1024x768 graphics in Version 5.3

VCAD53 800

for 16-color, 800x600 graphics in Version 5.3

VCAD54 1024

for 16-color, 1024x768 graphics in Version 5.4

VCAD54 800

for 16-color, 800x600 graphics in Version 5.4

VCAD54 256

for 256-color drivers in Version 5.4

For VersaCAD 5.4/386, edit the VCAD386.BAT file in the root

directory. Replace the name of the driver with the new driver.

SVGA Accelerator Software Drivers-25

WORDPERFECT

------------------------------------------------------------

This section contains information for WordPerfect Versions 4.2,

5.0, and 5.1.

Running WordPerfect 5.0 and 5.1 with Graphics Screen Driver

------------------------------------------------------------

Use the following instructions to configure WordPerfect 5.0 or

5.1 for the supported graphics screen drivers. The INSTALL

program will copy the graphics screen drivers for the selected

resolution into the directory where WordPerfect 5.0 or 5.1

(WP.EXE) is located.

WordPerfect can be configured for use with any of the following

graphic drivers:

Driver      Version         Resolution or   Number of Colors

Filename                    Columns x Rows

=============================================================

WDPVGA2.WPD WordPerfect 5.0 1024x768        16

WDPVGA1.WPD WordPerfect 5.0 800x600         16

WDPVGA3.VRS WordPerfect 5.1 1024x768        16

To complete the WordPerfect 5.0 or 5.1 configuration using the

selected graphics screen drivers, perform the following steps:

1.   Type:

WP

and press [Enter].

2.   Press [Shift+F1], the Setup key.

3.   Choose the "Display" option.

4.   Select the "Western Digital Imaging" driver.

SVGA Accelerator Software Drivers-26

Running WordPerfect 4.2 in 132-Column Text Modes

-----------------------------------------------------------------

Some programs such as WordPerfect 4.2 can use 132-column text

modes without any modification of the software. Simply tell the

software the number of rows and columns of text you can display

on your monitor, and the software will take advantage of the new

configuration.

1.   Use VGAMODE.EXE to switch to the 132-column mode of your

choice. Enter:

VGAMODE 13325

(to  switch to 132x25)

VGAMODE 13228

(to switch to 132x28)

VGAMODE 13244

(to switch to 132x44)

VGAMODE VGA

(to switch to standard, 80x25 text)

2.   Start WordPerfect with a /S after the name of the program to

get to the Setup Menu. Type:

WP/S

and press [Enter].

3.   Choose Option 3, "Set Screen and Beep Options."

4.   Enter the number of rows (25, 28, or 44) and the number of

columns (132). Press the [Enter] key until you return to the

Setup Menu.

5.   Choose Option 0 to accept the configuration and enter

WordPerfect.

6.   When you exit WordPerfect, run the command VGAMODE VGA to

return the screen to standard, 80x25 text mode.

SVGA Accelerator Software Drivers-27

Once WordPerfect is configured, you do not need to use the Setup

Menu again except to make configuration changes.

When in the WordPerfect program, you can set the margins

([Shift+F8]) as wide as 132 characters.

NOTE: You might want to incorporate all the commands needed to

start WordPerfect in 132-column text mode into a batch file so

that the process of starting the program will be automatic.

Example batch file contents:

VGAMODE 13244 [Enter]

WP [Enter]

CLS [Enter]

VGAMODE VGA [Enter]

WORD

-----------------------------------------------------------------

To configure Word 5.0 or Word 5.5 for use with the 800x600,

16-color driver, follow these steps.

Selecting the Video Driver for Word 5.0 and Word 5.5

-----------------------------------------------------------------

1.   Run the INSTALL program on the MS-DOS Applications Diskette.

2.   Choose the driver resolution, destination, and directory

name during the installation process.

3.   Consult your Word documentation for additional information

if you are running a special keyboard driver. Otherwise,

choose "Yes" to select the default keyboard driver.

4.   Rename the driver filename as shown on the INSTALL program.

(Change WDSCREEN.VID to SCREEN.VID.)

Word will now display in the enhanced resolution.

SVGA Accelerator Software Drivers-28

USING VESA.EXE

-----------------------------------------------------------------

VESA.EXE is a terminate-and-stay-resident (TSR) program that

enables your video circuitry to comply with Video Electronics

Standards Association (VESA) Version 1.2 for compatibility with

VESA drivers. VESA.EXE is an extension of the VGA BIOS video

services. Run it before running any VESA-compatible applications.

NOTE: The drivers for some applications designed to support high-

resolution graphics require VESA compatibility. The table of

Super VGA modes in the following section identifies some modes

that require you to run the VESA.EXE driver. Check your

application documentation for additional information on VESA

compatibility requirements.

To copy VESA.EXE to the hard disk, run the INSTALL program as

instructed for installing MS-DOS application drivers. At the

INSTALL main menu, select "Utilities." Then, select "VESA.EXE."

Follow the instructions on the screen.

To run VESA.EXE, type:

vesa

Then, press [Enter].

You can insert the VESA command in AUTOEXEC.BAT instead of typing

the command each time you start the application. Return to your

operating system documentation for information on AUTOEXEC.BAT.

SVGA Accelerator Software Drivers-29

VGA AND SUPER VGA PROGRAMMING

------------------------------------------------------------

This section describes how to access the Super VGA modes of the

VGA video controller. This information is intended for users

familiar with assembly-language programming. An understanding of

this information is not necessary for normal operation of the

video controller.

The VGA standard supports the video modes listed in the following

table. These modes can be accessed through standard video BIOS

calls from assembly language as well as high-level language

routines. When you start in MS-DOS, the computer is usually in

standard, 80-column text (alphanumeric) mode. On a color system,

this is Mode 3.

The SVGA Accelerator video controller supports the following

Super VGA modes in addition to the standard VGA modes:

.    1280x1024, 1280x960, and 1024x768 graphics

.    800x600 graphics

.    640x480 graphics in 256 colors

.    640x400 graphics in 16 or 256 colors

.    132-column text in 25, 28, 44, and 50 lines

Identification numbers for all the Super VGA modes available are

shown in "Super VGA Modes."

SVGA Accelerator Software Drivers-30

Standard VGA Modes

-----------------------------------------------------------------

Type   Video   Cols. Resolution Colors Char.  Buffer Mode Notes

Standard x Rows                    Size

T     CGA    40x25   320x200    16    8x8   B8000  0,1   1,2

T     EGA    40x25   320x350    16    8x14  B8000  0,1    1

T     VGA    40x25   360x400    16    9x16  B8000  0,1    1

T     CGA    80x25   640x200    16    8x8   B8000  2,3   1,2

T     EGA    80x25   640x350    16    8x14  B8000  2,3    1

T     VGA    80x25   720x400    16    9x16  B8000  2,3   1,3

T     MDA    80x25   720x350     4    9x14  B8000   7     1

T     VGA    80x25   720x400     4    9x16  B8000   7    1,4

G     CGA    40x25   320x200     4    8x8   B8000  4,5   1,2

G     EGA    40x25   320x200    16    8x8   B0000   D    1,2

G     VGA    40x25   320x200    256   8x8   A0000   13   1,2

G  Hercules  80x25   720x348   mono   9x14  B0000         1

G     CGA    80x25   640x200   mono   8x8   B8000   6    1,2

G     EGA    80x25   640x200    16    8x8   A0000   E    1,2

G     EGA    80x25   640x300     4    8x14  A0000   F     1

G     EGA    80x25   640x350    16    8x14  A0000   10    1

G     VGA    80x30   640x480   mono   8x16  A0000   11    1

G     VGA    80x30   640x480    16    8x16  A0000   12    1

1 T= text; G = graphics.

2 All 200-line modes are double-spaced to display 400 lines.

3 Default mode for color monitors.

4 Default mode for monochrome monitors.

SVGA Accelerator Software Drivers-31

Super VGA Modes

-----------------------------------------------------------------

Type Resolution Colors   Cols. x  Buffer  Mode    VESA    Notes

Rows            (Hex)   Mode

(Hex)

T   720x476     16      80x34    B8000   41       -       1

T   640x344     16      80x43    B8000   67       -       1

T   640x400     16      80x50    B8000   66       -       1

T   1188x400    16     132x25    B8000   55      109      1

T   1188x448    16     132x28    B8000   47       -       1

T   1188x396    16     132x44    B8000   54      10A      1

T   1188x396    16     132x44    B8000   21       -       1

T   1056x400    16     132x50    B0000   69      10B      1

G   640x400     256     80x25    A0000   5E      100      1

G   640x480     256     80x30    A0000   5F      101      1

G   800x600     16     100x75    A0000   58    6A,102     1

G   800x600     256    100x75    A0000   5C      103      1

G   1024x768    16     128x48    A0000   5D      104      1

G   1024x768    256    128x48    A0000   60      105      1

G   1280x960    16     160x60    A0000   6C       -       1

G  1280x1024    16     160x64    A0000   64      106      1

1 T= text; G = graphics.

SVGA Accelerator Software Drivers-32

Programming Considerations

-----------------------------------------------------------------

Programming the Super VGA modes is similar to programming the

standard VGA video modes of the IBM PS/2 VGA and PS/2 Display

Adapter.

Extended BIOS calls are provided to support the SVGA Accelerator

video controller's Super VGA modes. If you are writing to a

driver or an application that uses the Super VGA modes, use the

algorithms and tools described in the file PROGRAM.TXT. Read or

print this file for details on the recommended methods for

programming SVGA Accelerator video.

The 16-color 800x600 modes and 1024x768 modes require a

multiple-frequency monitor. The BIOS and hardware do not check to

determine if a multiple-frequency monitor is connected. The modes

are available regardless of the setting of the multiple-frequency

monitor timing switch on your video adapter or main logic board.

SVGA Accelerator Software Drivers-33

June 1992

The information in this document is subject to change without

notice and should not be construed as a commitment.

SVGA Accelerator Drivers and Utilities:

Copyright 1989-1992 (c) Western Digital Corporation. All Rights

Reserved.

SVGA Accelerator Utilities:

Copyright (C) 1990-1992 Tandy Corporation. All Rights Reserved.

Guide to SVGA Accelerator Software Drivers:

Copyright (C) 1992 Tandy Corporation. All Rights Reserved.

3D Studio, ADI, AutoCAD, AutoCAD 386, AutoShade, and RenderMan

are registered trademarks of Autodesk, Incorporated.

IBM, PS/2, and Personal System/2 are registered trademarks of

International Business Machines Corporation.

Lotus is a registered trademark of Lotus Development Corporation.

MS-DOS, Word, and Windows are registered trademarks of Microsoft

Corporation.

PCAD is a trademark of Personal CAD Systems, Inc.

Western Digital is a trademark of Western Digital Corporation.

1-2-3 and Symphony are trademarks of Lotus Development

Corporation.

Cadvance is a trademark of Isicad, Inc.

`

Generic CADD Level 3 is a trademark of Generic Software, Inc.

VersaCAD is a trademark of VersaCAD Corporation.

WordPerfect is a trademark of WordPerfect Corporation.