June 12, 1987

INSTALLING FOUR INTERNAL FLOPPY DRIVES IN AT-TYPE COMPUTERS.

Compiled by Jim Gainsley [76703,1007] From Technical Information and

Patches Provided By Clyde Washburn [70305,1211]

(CIS IBMHW Forum)

The PC/AT normally supports only two floppy drives. However, it is possible

to have four internal floppies, (or two external AND two internal), and

these may be any combination of the supported floppy drive types, e.g.

160kb/360kb/720kb/1.2mb, and 1.4mb.  You will not need to change any of the

controllers you have currently.

The following procedures may seem involved at first glance.  However, it's

one of those cases where it's more difficult to tell about than to

accomplish.

Hardware/Software:

1) IBM's External Drive Adapter, p/n #2683198, which can be purchased

through any IBM Dealer. Price currently approximates $110. and will

fit in a short slot.  Note that this is a smart switch and not a

controller.

2) A normal 34-pin two-floppy drive cable, except that it must be

designed for attachment to the External drive connector on the adapter,

i.e. having the 37-pin type D connector on the controller end. the cable

must have the IBM 'twist' on the floppy connector at the end of the cable.

(Note: The 34-wire flat cable is attached to the 37-pin connector such

that the stripe is towards the pin-1 end, but the cable is offset all

the way to the pin-37 end. Because of the way the pins are laid out,

wire-34 will actually go to pin-19 and wire-1 will actually go to

pin-21.)

If you're going to have this made, tell the supplier that it's going

to the external connector, so they'll make it a little longer -- about

6" -- than normal. This will make it a little easier to route.

3) The IBM external floppy software driver, EXDSKBIO.DRV. This has the

IBM p/n #6280666. However, you may find that you can obtain this for

free from your friendly IBM dealer... It comes with the IBM 3.5"

External drive adapter kit, (That kit includes the IBM external drive.)

4) Of course you'll need the floppies. For an internal 3.5x720k drive,

The TOSHIBA ND-353A/354A, with the internal mounting kit is suggested.

What to Do:

1) EXDSKBIO.DRV is coded for just one drive. In order to provide for

two drives, we make some patches and end up with two files. These are

then loaded as device drivers in the CONFIG.SYS files, as will be shown.

Make two copies of EXDSKBIO.DRV, naming them EXT1.DRV, and EXT2.DRV.

(Your 'original' unmodified EXDSKBIO.DRV should be put away for safe

keeping.)

Included in this ARC file are PATCHER.BAS, EXT1.PAT and EXT2.PAT.  Have

BASIC (or GWBASIC), PATCHER.BAS, EXT1.DRV, EXT1.PAT, EXT2.DRV, and

EXT2.PAT on the same disk, and in the default drive.  Now Boot BASIC

and run PATCHER twice. The first time enter EXT1.PAT when asked for

"patch file", and EXT2.PAT the second time.  If all went well, you will

now have two patched drivers EXT1.DRV and EXT2.DRV.

2) After the installation that follows, the two new drives will be

recognized AFTER ALL of your current physical and ramdrives. Place

the following in your CONFIG.SYS file, after any device drivers you

may have:

DEVICE=EXT1.DRV                        (Both MUST be loaded

DEVICE=EXT2.DRV                         BEFORE the DRIVER.SYS')

DEVICE=DRIVER.SYS  /d:2 /c              (For 3.5/720kb drives)

DEVICE=DRIVER.SYS  /d:3 /c              (For 3.5/720kb drives)

(Note:  If you've made the new drives 5.25x360kb's then the

DRIVER.SYS params would be: /D:2 /T:40 /F:0 & /D:3 /T:40 /F:0.

These additional params, T & F, are not needed for 3.5x720kb's

because those are the default for DRIVER.SYS.  The /c is necessary

for those drives that need the changeline feature. Changeline

is used for ALL drive types, EXCEPT 5.25x160/360kb.)

3) Turn off the computer and unplug it.  Remove cover.  Make a slot

available, preferably on the drive side of the floppy controller you

now have.

4) Remove the bracket from the External Adapter, (EA).  Cut out the side

of the bracket, the entire length of the connector slot, on the RIGHT

side, (top angle of bracket facing rear of computer, just as it will

be when installed).  Do this on an angle at each end, but don't disturb

the connector mounting holes.  Reconnect the bracket to the EA.

5) Thread the drive cable through the slot in the computer, stripe end

up.  Install the EA, being careful not to pinch the cable. The fit is

very close, but it will go.  Test by moving the cable up and down a

bit.  Be sure you feel at least a tiny amount of play. Connect the

cable to the EA.

6) Install the drives.  A jumper should be set to enable DC, (Disk Change)

IF the drives are ANY BUT 5.25x160/360kb units.  For those units,

Disk Change and Disk Ready, sometimes labeled DC or SR respectively,

MUST NOT be enabled (jumpered).

Disk Change is the 'changeline' feature mentioned in connection with

IBM's DRIVER.SYS documentation.  Disk Ready or Standard Ready or just

Ready, is NEVER used with an IBM PC/AT regardless of drive type!

Set a jumper for Drive Select line 2.  (DS2 where numbers go from 1 to 4

and DS1 where numbers go from 0 to 3), for BOTH units.  Also, for Toshiba,

be sure the jumper on the internal mounting kits pc connector board is

set for PC/XT, and NOT for AT.  For the Toshiba 3.5x720 then, install

jumpers on DC and DS2.

7) Connect the new floppy cable to the new drives. The connector at the

end of the cable must go to the drive you want to be first "logical"

drive of your new sequence. That is, if your new drives are going to

be say D & E, then D would be the FIRST "logical" drive, and would

take the connector at the END of the cable. (To say it another way,

the first logical drive is the one following the 'twist' in the cable.)

Note that the stripe end of the drive connectors goes to the right, (as

you face the front of the computer). Same side as any other floppies

already installed.

8) Power for the drives comes from the computers power supply through

the same power lead setup as powers the other internal floppy(s).

Your computer may have additional leads for this provision, or you

may have to have some "Y" cables made up to handle it.  Power does NOT

come from the 34-conductor cable.  Drives have frame grounding lug.

In most cases it is only necessary to ground the frame of one drive.

This usually serves to ground them all.  However, should you get

frequent drive errors, you should ground the affected drives.

9) At your floppy controller, note the orientation of the stripe on the

'internal' connector/cable for your current A:/B: drives. Then

disconnect it from the controller.

Take the short cable that came with the EA.  Attach one end to the

floppy controller where the A:/B: cable was, with the stripe oriented as

per the A:/B: floppy cable.  Attach the other end to the connector at

the 'front' end of the EA, stripe to the 'rear'.  Connect the A:/B:

cable to connector at the 'rear' of the EA, stripe to 'rear'.

10) You're done.  Boot the computer.  If formerly you had drives,

A/B as floppies and C as an HD, and D as a ramdisk, you will now

have additionally, E and F as your new drive designators.  As the

system boots, you should see the messages, "FIRST EXTERNAL DRIVE

LOADED" and "SECOND EXTERNAL DRIVE LOADED".

Test the setup first by reading something from each of your drives.

If all seems well, then write something to each.  Lastly, try

formatting a disk in each new drive, and run CHKDSK on them.

-END-

#######################################################