STANDARD MICROSYSTEMS CORPORATION

Novell NetWare OS/2 Workstation Requester Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This driver supports all 8216 (Ultra) and 80xx (PLUS) Series

ISA and Micro Channel Ethernet adapters.

This document explains how to:

o Use EZStart or a manual procedure to copy the NetWare OS/2

workstation requester driver from the SuperDisk.

o Install the driver using the Netware OS/2 requester INSTALL

program.

COPYING THE DRIVER FILES

EZStart Method:

Select 'Cancel' and then 'Copy Files' to copy all the

files used by this driver from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory that

you specify.

Manual Method:

The OS/2 Requester driver, SMC8000.SYS, and the NET.CFG file are

located in the OS2_ODI subdirectory on the SuperDisk. The driver is

installed using the NetWare INSTALL program.

LOADING THE DRIVER

Use the following instructions in conjunction with those in the

NetWare Workstation for OS/2 manual:

1.  Load the NetWare INSTALL program according to the

instructions in the NetWare manual.

2.  Select "Requester on workstation" from the menu.

3.  Specify the source and target directories.

4.  Select "Edit CONFIG.SYS..."

5.  Select "SMC8000.SYS" from the list.

If the driver is not on the list, enter SMC8000.SYS.

6.  Select or deselect any remaining options, then select

"Save". In the "Save File As:" field, type "CONFIG.SYS".

7.  In the "Copy Requester Files" window, select "Copy".

8.  Proceed with the instructions in the NetWare manual until

the installation is complete.

EDITING THE CONFIG.SYS FILE

1.  Copy the file SMC8000.SYS from the OS2_ODI subdirectory of

the SuperDisk to the NETWARE subdirectory on the workstation.

2.  Save the file, then reboot the computer.

INSTALLING MULTIPLE ADAPTERS

If two adapters are installed, you will need to modify the

NET.CFG file, located in the OS2_ODI subdirectory of the SuperDisk,

and copy it to the SMCNOV or the directory where the SMC OS2 ODI

driver is located.  To modify the file, use any text editor to

open the file and add the following lines:

LINK DRIVER SMC8000

PORT XXX (ISA adapters)

SLOT X   (Micro-channel adapters only)

LINK DRIVER SMC8000

PORT XXX (ISA adapters)

SLOT X   (Micro-channel adapters only)

(where XXX is the I/O address of the adapter in hexadecimal)

The first adapter should be the one with the lowest I/O base address

(use EZStart to obtain each adapter's I/O base address).

ERROR MESSAGES

Here is the list of error messages the driver may display:

"Adapter hardware failure detected (initialization)."

The adapter did not pass the initialization process.

Check the adapter for configuration conflicts.

"Adapter hardware failure detected (self-test)."

The adapter did not pass the initialization self-test process

due to a hardware problem. Check the adapter for configuration

conflicts.

"Adapter unable to insert into network."

The adapter did not connect to the network due to a hardware

error.  Check the adapter for configuration conflicts.

Check the adapter is inserted properly into the slot connector.

"Adapter IRQ higher than 7 not valid in 8-bit slot."

The I/O base address specified cannot be used if the adapter

is inserted into an 8-bit slot.  Select a different IRQ number.

"No Ethernet Adapters Found."

The driver could not detect an adapter installed in the system.

Check the adapter is inserted properly into the slot connector.

"Invalid Ethernet node address specified."

The node address specified in the NET.CFG file was illegal.

Broadcast and multi-cast addresses are not allowed as valid

node addresses.  Use the node address over-ride with caution

as this can cause problems on your network.

"One or more NET.CFG parameters are missing or invalid."

A driver parameter required to load the driver is missing from

the NET.CFG file.  Add the parameter(s) and re-load the driver.

"I/O address in NET.CFG is on illegal boundary."

The specified PORT address in the NET.CFG file is not correct.

Verify the adapter I/O address from the soft-configuration or

jumper setting and make the necessary changes to the NET.CFG

file.

"I/O address in NET.CFG is out of range."

The I/O base address specified in the NET.CFG file for the

AT-bus adapter is out of range of supported values.

Verify the adapter I/O address from the soft-configuration or

jumper setting and make the necessary changes to the NET.CFG

file.

"IRQ in NET.CFG does not match adapter configuration."

The IRQ number specified in the NET.CFG file for the

AT-bus adapter does not match the detected configuration.

Verify the adapter IRQ setting from the soft-configuration or

jumper setting and make the necessary changes to the NET.CFG

file.

"IRQ specified in NET.CFG is out of range."

The IRQ number specified in the NET.CFG file for the

AT-bus adapter is out of range of supported values.

Verify the adapter IRQ setting from the soft-configuration or

jumper setting and make the necessary changes to the NET.CFG

file.

"RAM address in NET.CFG is on illegal boundary."

The specified MEM address in the NET.CFG file is not correct.

Verify the adapter RAM memory address from the soft-configuration

or jumper setting and make the necessary changes to the NET.CFG

file.

"RAM address in NET.CFG is out of range."

The RAM address specified in the NET.CFG file for the

AT-bus adapter is out of range of supported values.

Verify the adapter RAM memory address from the soft-configuration

or jumper setting and make the necessary changes to the NET.CFG

file.

"RAM size specified in NET.CFG is out of range."

The RAM size specified in the NET.CFG file for the AT-bus adapter

is out of range of supported values. Verify the adapter RAM

memory address from the soft-configuration or jumper setting and

make the necessary changes to the NET.CFG file.

"RAM size in NET.CFG does not match adapter config."

The MEM length parameter in the NET.CFG file was invalid.

Check the adapter configuration and make the necessary changes

to the NET.CFG file.

"Cannot access adapter RAM properly."

The adapter could read or write the adapter RAM.  Verify that

another adapter does not share the same memory space as the

LAN adapter.

EtherCard Ultra, EtherCard PLUS, TokenCard, and SuperDisk are

trademarks, and SMC and Standard Microsystems are registered

trademarks of Standard Microsystems Corporation. Other Trademarks

mentioned herein belong to their respective companies.