THE MOUSE SYSTEMS CORPORATION

MSCMOUSE DRIVER version 6.23

Your Mouse Systems Software package contains two Mouse drivers:

MSCMOUSE.SYS and MSCMOUSE.COM.  The MSCMOUSE.SYS mouse driver can only be

used in the CONFIG.SYS file where the MSCMOUSE.COM can be used either

in your AUTOEXEC.BAT or anytime at the DOS prompt.

COMMONLY USED COMMAND LINE OPTIONS

The following are common command line parameters to be used with the mouse

driver.  Enter command:

MSCMOUSE <arg>

where <arg> represents one or more of the following:

/1    Direct Mouse Driver to COM1

/2    Direct Mouse Driver to COM2

/C    Direct Mouse Driver to Bus Card

/P    Direct Mouse Driver to PDP port (IBM PS/2)

/A0   Disable acceleration and set resolution back to the default

/A#   Invoke High-Resolution dynamic acceleration level (1 - 9)

/E    Enable/Disable resolution doubling

/I    Enable/Disable application defined mouse sensitivity

/U    Unload MSCMOUSE and POPUP drivers to release memory

/?    Exercise help parameter list

For example, the following commands will direct the mouse driver to COM2, set

the mouse acceleration to 8, set the base resolution to 200-DPI (assuming a

100-DPI mouse), and disable an application program's ability to set its own

mouse sensitivity:

1. In the AUTOEXEC.BAT (or at the command line/DOS prompt):

MSCMOUSE /2/A8/E/I           (NOTE!! Could also be: mscmouse /2/a8/e/i)

or

\MOUSE\MSCMOUSE /2/A8/E/I

(If you copy MSCMOUSE.COM to a subdirectory called MOUSE)

2. In the CONFIG.SYS:

DEVICE=MSCMOUSE.SYS /2/A8/E/I

or

DEVICE=\MOUSE\MSCMOUSE.SYS /2/A8/E/I

(If you copy MSCMOUSE.SYS to a subdirectory called MOUSE)

SETTING MANUAL ACCELERATION

To set mouse acceleration manually, press CTRL + ALT + Left Mouse Button.       After the beep, release the three keys and press any number 0 - 9 on the

keyboard.  If you do not make a valid selection within 6 to 8 seconds, the

mouse driver will beep three times and return you to your application with

the mouse set as before.  Otherwise, there will be two beeps and you will

return to your application with a new acceleration level.

PROBLEMS

If you are experiencing any problems with either your Mouse Systems mouse

or the software, please contact us at:

MOUSE SYSTEMS CORPORATION

47505 SEABRIDGE DRIVE

FREMONT, CA 94538

PHONE: (415) 656-1117

FAX: (415) 770-1925

BBS: (415) 683-0617