PDU V3.0

(c)2002 Stefan Peichl, Heidelberg, Germany

http://home.t-online.de/home/stefan.peichl

eMail: stefan.peichl@t-online.de

PDU.COM converts an ASCII short message (SMS) into a protocol

data unit (PDU) and vice versa. PDU format should work with

every mobile phone supporting SMS, but it is the only choice

for some phones like the Siemens S25 or S35 for example.

If you want to handle your SMS like email on the HP200LX, you

should use the POST/LX plugin POSTPDU, written and maintained

by Tony Hutchins. Please download from:

http://www.geocities.com/aghutchins/postpdu.zip

REVISION HISTORY

Version 3.0  -Separate POST/LX plugin POSTPDU.ZIP

(20-JAN-02)  -Bug fix for odd SMSC number length

-Beep only for >160 character instead of >=160

-Trailing CR LF ignored for 160 character count

-Wired serial communication for S25 removed

Version 2.6  -Improvements by Tony Hutchins:

(26-DEC-01)  -Allows configurable font for PDU.SCR

-IrDA now kept alive for very large outbox file

(N.B. this requires ROBOT version >=1.8)

-Changed screen display to include a status line

-Cleanup of temporary files made optional.

-PDU.BAT now handles ROBOT.CFG usage automatically

Version 2.5  -Improvements by Tony Hutchins:

(01-DEC-01)  -Removed dependence on directories in the setup

-Allow optional usage of ROBOT.CFG

-Message ref. number can be recorded for sent SMS

-More than one SMS box allowed

Version 2.4  -Bug fix for odd destination number length

(14-NOV-01)  -Use of POST.CFG instead of ROBOT.CFG

-ROBOT error report as incoming SMS

-Automated IrDA and Baud setup in PDU.BAT

Version 2.3  -Use of 8086 opcodes instead of 80186

(14-SEP-01)

Version 2.2  -SMS transport layer date stamp bug fixed

(05-SEP-01)  -SMSC selection in POST/LX through Sender: entry

in PDU.TPL instead of SMSC: entry

Version 2.1  -Phone Numbers may now contain the chars: ()/.-

(24-APR-01)  -PDU.LEN is now correct for NOTA-GSM.

Version 2.0  -Year is now reported with 4 digits instead of 2

(21-NOV-00)   for sorting reasons in POST folders.

Version 1.9  -MIME encoded end of lines of POST are removed.

(18-NOV-00)

Version 1.8  -Norwegian characters added.

(01-SEP-00)  -Support of Ericsson I888 using the /A switch

for downloading SMS from the phone.

-eXcell gateway number updated in PDU.SCR.

-PDU.SCR bug fixed (download of send/unsend SMS).

Version 1.7  -More international characters added.

(09-JUL-00)

Version 1.6  -Flash SMS selection with 1.char '!' of the SMS.

(23-JUN-00)  -Automatic SMS to Email convertion using the eXcell

gateway, if "To:" is an email address.

-Useless SMS class0-3 selection removed.

Version 1.5  -Switch /B added to disable default baud rate

(14-JUN-00)   setting of 19200 for stand alone use of PDU.COM

-Robot script PDU.SCR now also downloads SMS

stored in the ME memory of the mobile.

-Unreliable timezone display removed.

-SMS class0-3 selection with 1.char of the SMS.

Version 1.4  -Better decoding of Originater Phone number

(09-MAY-00)

Version 1.3  -Support of IrDA Nokia phones with the included

(03-MAY-00)   ROBOT script PDU.SCR

Version 1.2  -Nokia NOTA-GSM support with /A switch

(31-MAR-00)

Version 1.1  -Decoding of "...expired..." Status message

(13-DEC-99)

Version 1.0  -First release

(04-DEC-99)

HOW TO USE PDU

PDU searches by default for the file SMS.TXT in the local

directory and converts it to SMS.PDU. Indeed you may use any

other filename with the extension TXT from the command line.

PDU detects the file extension and acts upon it. A TXT file is

converted to a PDU file of the same name, and a PDU file is

converted to a TXT file of the same name. The corresponding

files are always saved in the same directory.

The TXT file must use a simple format:

On the first line of the TXT file, you may provide an optional

SMSC (Short Message Service Center) of the form

SMSC: 1234567

If you omit the first line, the SMSC is used, which was stored

in your mobile by a previous AT+CSCA command.

On the second line of the TXT file, or on the first line, if

no SMSC is present, you may enter the number of the recipient

of the SMS. The "To:" is mandatory

To: 1234567

If you omit the To: line, no destination number is filled into

the SMS, and you have to select a number from the phone book of

your mobile, before you send out the SMS. This is only useful

if you want to copy the SMS to the SIM and send it out later

using an entry of your phone book.

All following lines of the TXT file should hold your message.

You may use up to 160 characters for your message. Exceeding

characters are truncated, and you are informed by a beep and

a text message and an errorlevel of this truncation.

If you want to use international phone numbers for either the

SMSC or the destination, your number must start with the '+'

character. Within the number you may use blancs and /-().

-EXAMPLES-

-SMS with SMSC and destination in international format:

SMSC: +49 176 0000442

To: +49 172 1234567

Your message goes here...

-SMS with national destination number. SMSC from mobile is used

To: 0172-1234567

Your message goes here...

-SMS without destination but national SMSC

SMSC: 1760000442

Your message goes here...

-SMS without destination and without SMSC

Your message goes here...

A delivered PDU is converted to a TXT short message of the form

SMSC: +491760000442

From: +491721234567

Date: 27 Nov 99 12:50:31

The delivered message goes here...

A status report is converted to the form

SMSC: +491760000442

From: +491721234567

Date: 27 Nov 99 12:50:31

Message 023 delivered 28-Nov-99 15:37:05

PDU.COM reports several errorlevels for use in batch files:

0 = error free execution

1 = file error (not found, cannot create, zero length...)

2 = text truncated to 160 characters

3 = illegal character in PDU file

PDU/? offers a short help screen.

COMMAND LINE SWITCHES

Most of the switches are only needed in conjunction with the

use of PDU.COM in a ROBOT script.

Switches for converting TXT to PDU:

/R = Status Report Request

/M = use ISO-8859-1 Quoted printable (ROBOT)

Switches for converting PDU to TXT:

/J = Add a "Subject:" Line to SMS.I  (ROBOT)

Switches for both directions:

/N   = No Informational Display      (ROBOT)

/A   = Use NOTA-GSM mode             (ROBOT)

/I   = ISO-8859-1 8-Bit coding       (ROBOT)

STATUS REPORT REQUEST & FLASH SMS

PDU.COM allows two ways to request a status report for a send

out SMS. You may either use the command line switch /R or you

may write the '+' character as the first character of your

message text. If you put the '-' char at the first position in

your text, a previously set command line switch /R is ignored

and no status report is requested. PDU.COM swallows the '+' or

'-' and does not transmit it to the receiver of the SMS.

If your first character of the SMS text is a '!', then the SMS

is transmitted as flash SMS and should be immediately displayed

on the receivers phone.

SENDING OUT THE PDU

In order to send out a PDU to the mobile phone, you need a

communications program which is able to talk to the phone

through the serial port or an IrDA interface. To simplify

things, the communications program should have script

capabilities to automate the upload of a PDU to the phone.

First you have to setup the correct baud rate for the phone.

Then execute the appropriate GSM 07.05 AT command:

AT+CMGW=140    ;copy SMS to SIM

AT+CMGS=140    ;send out SMS to GSM

After you executed the command you have to send out the PDU

file with the send file option of your communications program.

After the file has been sent, you have to enter the required

Ctrl-Z character manually to terminate input.

If you have no success with this procedure, try to use

instead of the maximal byte length 140 the actual byte length

provided in the file PDU.LEN after every conversion. NOKIA

phones require the actual length rather than the maximum.

Once everything works, you should write a script to automate

the whole procedure. A professional example of a script for the

communications program WWW/LX is found in POSTPDU.ZIP mentioned

at the beginning of this documentation.

DEFAULT CONFIGURATION VALUES

PDU does not allow configuration of every supported feature of

protocol data units. The following features are fixed:

-no reply path is used

-no user data header support

-no 'reject duplicate' support

-validation period is fixed to one week

-7 bit data is used (hence 160 characters text)

However decoding of 7 and 8 bit data is supported as well as

translation of german 'umlaute' under plain DOS and under

POST/LX using either ISO-8859-1 8bit or quoted printable

representation.

"Delivered" and "Expired" Status Reports are decoded by PDU.COM.

DISCLAIMER

This software is copyrighted freeware.

The author disclaims all warranties, expressed or implied.

The author assumes no liability for damages, direct or

consequential, which may result from the use this software.