AI-LANBIOS Low-Level Driver For AMD-Compatible Adapters (PCNTLC.DOC)

--------------------------------------------------

(C) Copyright 1993 ARTISOFT, Inc.

(Revised 3/2/93)

INTRODUCTION

============

The PCNTLC.EXE low-level AI-LANBIOS driver allows you to use your Novell

NE2100, NE1500T or AMD 2100 compatible adapters in a LANtastic network.

This includes AMD's PCnet-ISA architecture which is compatible with

the 2100.

In order for a Novell-compatible NE2100/1500T adapter to function with

the LANtastic Network Operating System (NOS), you must first install the

adapter as instructed in your adapter installation guide, then run the

supporting software (PCNTLC.EXE and AILANBIO.EXE).  PCNTLC.EXE communicates

with the adapter, while AILANBIO.EXE provides high-level communications

across the local area network.  Because of this, you cannot run any

network software until after you load PCNTLC.EXE and AILANBIO.EXE.

(Note: For information on running AILANBIO.EXE, refer to AILANBIO.DOC

on your LANtastic driver diskette.)

PCNTLC.EXE V4.0

=============

There are command line switches that you can use when you run PCNTLC.EXE

and AILANBIO.EXE.  These switches allow you greater flexibility in using

these programs.  For information on the AILANBIO.EXE command line options

refer to AILANBIO.DOC on your LANtastic driver disk.  For information on

PCNTLC.EXE command line switches, refer to "PCNTLC.EXE COMMAND LINE OPTIONS"

in this file.

RUNNING PCNTLC.EXE

================

The syntax for running PCNTLC.EXE is

PCNTLC[switches . . . ] [ ; comment]

where "switches" denotes any optional command line switches.  If you are

using more than one switch in a command line, use a space or a forward

slash (/) to separate the arguments.  If a switch takes a value (such as

when you designate the DOS multiplex number with the MPX= switch) the

switch must be followed by an equal sign (=) or a colon (:).  All switch

values are checked to make sure they are valid.  Illegal values produce

error messages.

These are valid switch formats:

SWITCH

/SWITCH

SWITCH:VALUE

/SWITCH=VALUE

You would use one of the first two formats for a switch that does not take

a value.  For example:

PCNTLC/HELP

You would use the third or fourth formats for a switch that takes a value.

Such as:

PCNTLC/MPX=D7

PCNTLC.EXE COMMAND LINE OPTIONS

=============================

The valid PCNTLC.EXE switches appear below.  The letters "ddd"  denote

that you must provide a decimal number.  The letters "hh" denote a

required hexadecimal value.  Values enclosed in brackets "[]" indicate

default values if the switch is omitted.  Values after the brackets denote

the valid range of variables for the switch.

----------

@switch-file

This option specifies that further switches are to be taken from a

switch file.  Any switches after the @switch-file name will be used,

and the switch settings take precedence over any settings in the switch

file.  You may invoke switch files from within switch files as often as

you wish.  The file should contain valid switches and may contain comment

characters (;) at the beginning of each line or after switches.

For example,

PCNTLC @setup

The file SETUP contains

; PCNTLC AI-LANBIOS driver setup file

;

IOBASE=360            ; Use I/O port address 360

IRQ=4                 ; Use interrupt request 4

VERBOSE               ; Display verbose information

----------

HELP or ?

This switch instructs PCNTLC.EXE to display information about the valid

command line switches, but does not install the PCNTLC.EXE driver.  For

example,

PCNTLC/help

PCNTLC ?

See the messages section for a detailed description of the HELP display

output.

----------

IEEE

This switch causes all transmitted packets to be in the IEEE 802.3

standard instead of the XEROX format.

----------

IOBASE=hhh [300] One of the following choices:

300 320 340 360

This switch specifies the I/O base address configured on the adapter.  The

IOBASE switch value must match the jumper setting on the card.  The

default value is the same as the default factory setting on the cards.

The IOBASE numbers must be in multiples of 20H.  If a non-multiple is

encountered, the I/O base address is rounded down to the closest address.

For example, IOBASE=310 is the same as IOBASE=300.

Consult your adapter installation guide for more information about setting

the I/O base address on the card.

----------

IRQ=dd [5] 3-15

This switch specifies the interrupt request (IRQ) level for the board.  If

you have more than one adapter installed, each should have its own unique

IRQ.  The IRQ switch value must match the jumper setting on the card.  The

default value is the same as the default factory setting on the card.

Consult your adapter installation guide for more information on setting

IRQ jumpers on the cards.

NOTE: If you have a 2100 compatible adapter, not all interrupts are

supported. These adapters support only interrupts 3,4,5 and 9.

----------

DMA=d [5] 3-7 (Excluding 4)

This switch specifies the DMA channel (DREQ/DACK) for the board. If you

have more than one adapter installed, each should have it's own unique

DREQ/DACK setting. The DMA switch value should match the jumper setting

on the board.	Both the DREQ and DACK jumpers must be set to the same number.

The default number is the same as the default factory setting on the card.

Consult your adapter installation guide for more information on setting

DREQ/DACK jumpers on the cards.

NOTE: DMA level 4 is used to select a special case of no DMA enabled at all.

This is used to support future versions of adapters. On adapters which

support DMA the selftest will fail if DMA level 4 is selected.

----------

MPX=hh [C7] C0-FF

This switch specifies the DOS multiplex (MPX) number to use for

communication with AILANBIO.EXE.  You will need to change this value only

if the default MPX number is in use by another application or NETBIOS.

Multiplex numbers 00 through BF are reserved for DOS use; therefore, you

must use a multiplex number between C0 and FF.

----------

PACKET_SIZE=dddd [1500] 570-1500

This switch specifies the maximum size of transmitted IEEE 802.3 packets.

You may need to change this value when using certain bridges that do not

support the full 1500-byte packet size.  Even when you reduce the packet

size, the low-level driver can still receive larger packets.

----------

RBUFS=dd [4] 2-16 (2,4,8,16)

This switch specifies the number of allocated receive buffers used by

PCNTLC.EXE. Increasing this number can increase performance, but will also

require the use of more memory.

Receive buffers must be a power of two. Any other number will be rounded

down to the nearest power of two.

For example, RBUFS=7 is the same as RBUFS=4.

----------

TBUFS=dd [2] 2-8

This switch specifies the number of allocated transmit buffers used by

PCNTLC.EXE. Each transmit buffer occupies 1520 bytes, so increasing this

number will increase memory usage accordingly. Increasing this number

can increase performance of the network.

----------

REMOVE

This switch removes PCNTLC.EXE from memory.  You must remove TSR programs

in the opposite order they were loaded.  For example, if you loaded

PCNTLC.EXE and then AILANBIO, you must remove AILANBIO before PCNTLC.EXE

In addition, if you have more than one invocation of PCNTLC.EXE and

AILANBIO you must use the MPX= switch on the command line to specify which

copy of PCNTLC.EXE but not (AILANBIO) that you want to unload.  For

example, if you load two copies of PCNTLC.EXE and AILANBIO:

PCNTLC.EXE/MPX=C7

AILANBIO/ADAPTER=0/MPX=C7

PCNTLC.EXE/MPX=D7

AILANBIO/ADAPTER=1/MPX=D7

To unload these copies of PCNTLC.EXE and AILANBIO:

AILANBIO/REMOVE

PCNTLC.EXE/MPX=D7/REMOVE

AILANBIO/REMOVE

PCNTLC.EXE/MPX=C7/REMOVE

The REMOVE option will remove the most recently run copy of AILANBIO.

----------

VERBOSE

This switch causes the PCNTLC.EXE driver to be installed and detailed

information about the configuration of the PCNTLC.EXE driver to be

displayed.  See the messages section for a detailed description of the

VERBOSE output.

----------

XBUS

This switch causes the PCNTLC.EXE driver to extend the bus timing of a

PCnet-ISA based adapter. This allows operation in certain machines

that require longer bus cycles such as the Toshiba T5200 series

laptops.

USING MULTIPLE ADAPTERS

=======================

IMPORTANT NOTE: Multiple 2100 style adapters may not operate in some

machines due to the bus master architecture of the design in relation

to the motherboard timing.

PCNTLC.EXE and AILANBIO.EXE communicate with each other using a DOS

multiplex (MPX) number.  Each PCNTLC.EXE and AILANBIO.EXE pair supports

one adapter.  You will need to run an additional copy of PCNTLC.EXE and

AILANBIO.EXE for each additional adapter installed in your computer.

Using command line switches, you must assign each PCNTLC.EXE and

AILANBIO.EXE pair a unique multiplex and adapter number.  You must also

make sure each copy of PCNTLC.EXE has a unique IRQ line, DMA channel and

I/O port address.  For example:

PCNTLC

AILANBIO

PCNTLC/MPX=D7/IRQ=4/DMA=6/IOBASE=320

AILANBIO/MPX=D7/ADAPTER=1

The first invocation of PCNTLC.EXE and AILANBIO.EXE loads into the first

adapter using the default MPX number (C7), interrupt request number (5),

DMA channel (5), I/O port address (300) and adapter number (0).  The

command line switches in the second invocation instruct the NETBIOS to

load into the second card, (adapter number 1) using interrupt request 4,

DMA channel 6, IOBASE 320 and DOS MPX number D7.

PCNTLC.EXE MESSAGES

===================

The PCNTLC.EXE driver produces two types of messages: Informative and

Error.  All error messages are preceded by the text "ERROR:" and in those

cases the PCNTLC.EXE driver is not installed.

--------

MESSAGE:

AI-LANBIOS PCNTLC driver Vn.nn - (C) Copyright 1992 ARTISOFT Inc.

This message is displayed each time the low-level driver is invoked.

--------

MESSAGE:

---- PCNTLC driver installed ----

This message is displayed after the low-level driver is successfully

installed.

--------

MESSAGE:

---- PCNTLC driver NOT installed ----

This message is displayed if the low-level driver is not installed.  The

driver will not be installed if HELP information is displayed or an error

occurred.

--------

MESSAGE:

Command line              /verbose

IEEE 802.3 node address  00001A188085   Network packet size      1500

Allocated Rx buffers     4              Allocated Tx buffers     2

MPX interface number     C7             IO base address          0300

Interrupt request (IRQ)  5              DMA channel (DREQ/DACK)  5

Packet type              XEROX          Bytes of memory used     11648

This message is displayed if the VERBOSE switch is specified.  The values

displayed specify how the low-level driver is configured.  Some values may

not agree with switch values specified on the command line.  This will

happen when you specify an IOBASE switch value that is not a proper

multiple of the allowed value, as an example.

--------

MESSAGE:

Valid command line switches:

HELP

IEEE

IOBASE= Range 0300 to 0360 hex

IRQ= Range 3 to 15 decimal

DMA= Range 3 to 7 decimal

MPX= Range C0 to FF hex

PACKET_SIZE= Range 574 to 1500 decimal

RBUFS= Range 2 to 16 decimal

TBUFS= Range 2 to 8 decimal

REMOVE

VERBOSE

XBUS

?

@ Range 0 to 2048 byte file

This message is displayed when the HELP or ? switch is used.  Each valid

switch is listed with the acceptable range of values (if any) and the base

(hex or decimal) that the numbers must be entered in.  The acceptable file

size range for indirect files (@) is also listed.

--------

MESSAGE:

PCNTLC not loaded

You have issued the PCNTLC/REMOVE command and do not have PCNTLC.EXE

loaded into memory.  Only issue this command after the program is loaded.

-------

MESSAGE:

---- PCNTLC removed ----

The PCNTLC.EXE driver has been removed from memory.

-------

ERROR:    Can't REMOVE -- PCNTLC interrupts re-hooked

Meaning:  You have attempted to remove PCNTLC.EXE and the interrupts are

in use by another TSR, or PCNTLC.EXE was not the last TSR

loaded.

Remedy:   Remove any TSRs loaded after PCNTLC.EXE.  For example, if you

load PCNTLC.EXE then AILANBIO.EXE, you must first remove

AILANBIO.EXE, then you can remove PCNTLC.EXE.

-------

ERROR:    Installed PCNTLC is different version

Meaning:  You have loaded a version of the PCNTLC.EXE program and

attempted to remove it from memory with the /REMOVE switch.  The

version with which you are attempting to remove PCNTLC.EXE is

not the same version that was loaded.

Remedy:   Remove PCNTLC.EXE with the same version of the program that was

loaded.

------

ERROR:   Cannot open indirect file - @file

Meaning: The file "file" cannot be opened as an indirect file.

Remedy:  The file must exist before it can be used as an indirect file.

------

ERROR:   Illegal character after switch name - ??????xnnnn

Meaning: The switch ?????? is followed by a character other than "=" or

":" or the switch does not take a value.

Remedy:  Only use the characters "=" or ":" to specify switch values and

do not place characters after switches that do not take values.

------

ERROR:   Illegal digit in switch value - ??????=nnnn

Meaning: An illegal digit (character) was encountered in the numeric

switch value.

Remedy:  You must restrict numeric decimal values to the numbers 0-9.

You must restrict hexadecimal values to the numbers 0-9 and the

letters A-F.

------

ERROR:   Illegal switch - ??????=nnnn

Meaning: The switch ??????=nnnn is not a recognized switch.

Remedy:  You may only specify legal switches on the command line.

------

ERROR:   MPX number hh is already in use - Try another number

Meaning: The multiplex (MPX) interrupt number hh is being used by another

application or low-level driver.

Remedy:  Try another MPX number.

------

ERROR:   No network hardware found at I/O base address hhhhH

Meaning: PCNTLC.EXE did not find an adapter at the I/O port address

specified on the command line.

Remedy:  Make sure the adapter is correctly installed and that the IOBASE

command line switch corresponds to the I/O base jumper setting on

the board.

NOTE:    This message will also appear if the DREQ jumper is missing from

the adapter.

------

ERROR:   Insufficient memory to load driver

Meaning: Not enough free DOS memory existed to load the driver.

Remedy:  Remove other resident programs to free conventional memory.

------

ERROR:   VDS memory allocation failed

Meaning: An error occurred trying to allocate a physical DMA region.

Remedy:  Load driver in conventional memory or check memory manager.

------

ERROR:   Adapter self test failed -- <sub message>

Meaning: The PCNTLC performs a self test when the driver is loaded and

has failed.

Remedy:  Replace the adapter or if a PCnet-ISA type card is being

installed, set the XBUS command line switch.

Valid sub messages:

Initialization timeout

Loopback timeout

Network error

Transmit error

Receive error

Message compare error

------

ERROR:   Incorrect DMA jumper setting

Meaning: The jumper setting of the adapter doesn't match the value

given on the command line.

Remedy:  Make sure the command line DMA switch matches the DREQ/DACK

jumper setting of the adapter.

------

ERROR:   Incorrect IRQ jumper setting

Meaning: The jumper setting of the adapter doesn't match the value

given on the command line.

Remedy:  Make sure the command line IRQ switch matches the IRQ jumper

setting of the adapter.

------

ERROR:   Switch value not in range - ??????=nnnn

Valid range is ssss to eeee

Meaning: The value nnnn is not in proper range.  The value "ssss"

represents the lowest acceptable value and the value "eeee"

represents the highest acceptable value.

Remedy:  Limit your range of values to the acceptable range.

TESTING PCNTLC.EXE RETURN CODES (ERROR LEVELS)

==============================================

You may test the PCNTLC.EXE return code or error level in a batch file by

using the IF ERRORLEVEL command.  The following error levels are returned:

0     No error encountered.  PCNTLC.EXE driver installed

1     MPX number is in use

2     Switch error

3     No network hardware present

4     PCNTLC not loaded

5     Can't remove, different version loaded

6     Cannot remove, interrupts re-hooked

7     ERROR: Can't REMOVE -- AI-LANBIOS (R) still installed

9     Insufficient memory to load driver

10    Incorrect DMA jumper setting

11    Incorrect IRQ jumper setting

12    Adapter failed self test

13    VDS memory allocation failed

------------------

AMD is a registered trademark of Advanced Micro Devices, Inc.

PCnet-ISA is a trademark of Advanced Micro Devices, Inc.

Novell is a registered trademark of Novell, Inc.

#